CUMULUS LINUX BOOTCAMP: LAB GUIDE

Scope

This workbook covers configuration of network protocols on Cumulus Linux Network Operating System.

Audience

This workbook is intended for Technical Training students.

Objectives

By the end of this workbook, students will be able to:

- Configure basic switch functions with Cumulus Linux

- Configure layer 2 and layer 3 protocols with Cumulus Linux

- Verify configuration and connectivity

- Monitor and troubleshoot networking related connectivity issues

Overview

Each student will be using the Cumulus Air © platform, exercises in this workbook on a group of devices (servers and switches).

Notice

Please follow the instructions below carefully to successfully complete the practice. 
If you encounter technical issues, please contact the Nvidia Networking Academy team: 
nbu-academy-support@nvidia.com

Release Date

Revision 2.3 – February 2021

Good Luck, 
NVIDIA Networking Academy team

TABLE OF CONTENTS

| Prerequisites and Guidelines……………………………………………….……………………….….... | 3 |
| --- | --- |
| Academy Lab Topology ....…………………………….………………………………………………….... | 4 |
| Academy Lab Access………………………………….………………………………………………………. | 5 |
| PRACTICE UNITS |  |
| Practice 1: Using the NCLU ……………………………………………………………………..…...….. | 6 |
| Practice 2: Basic Switch Functions ……….…….……………………………..………..……….…… | 10 |
| Practice 3: VLANs and Trunking ……………………………………………………………..…….…… | 15 |
| Practice 4: Configuring MLAG and VRR …………….……………………….……………..….……. | 21 |
| Practice 5: Configuring BGP Unnumbered……….………………………………….…………….. | 32 |
| Practice 6: Configuring VXLAN with EVPN ………………….………………………….………….. | 40 |
| Practice 7: Configuring Distributed Asymmetric VXLAN Routing ….………….………… | 44 |
| Practice 8: Configuring Distributed Symmetric VXLAN Routing …………………………. | 47 |
| Practice 9: Editing Ansible Inventory ….………….……………………………………………..…… | 51 |
| Practice 10: Creating an Ansible Playbook …………………………………….…………..……… | 59 |

# PREREQUISITES AND GUIDELINES

Please perform and review the following steps before you start:

- Enter the Cumulus Air web page : https://air.cumulusnetworks.com/Login
Click “GET STARTED” button.

- If you have already created an account, 
use your credentials to Login.

- To sign up for the first time, click “Register” 
and fill in your details.

Once completed, a confirmation email will 
be sent, open it to activate your new account.

- Once you are logged in, you will reach the “Cumulus in The Cloud” dashboard. 
Wait for the lab to be Loaded.

- Click on the “Academy ILT” label.

## ACADEMY LAB TOPOLOGY

The training lab is organized in the following topology:

## ACADEMY LAB ACCESS

Click on a NODE to open its console

- When the login prompt appears, enter the username – “cumulus”

- When the password prompt appears, enter the password – “Academy123” and press Enter.

- You should now be prompted with the node’s name. This indicates that you have successfully accessed the node.

## PRACTICE 1: USING THE NCLU

Practice Objectives:

In this practice session you will become familiar with the Cumulus Linux NCLU:

- You will use ‘net add’ and ‘net del’ commands to change the configuration

- You will use ‘net commit’ command to apply configuration changes

- You will use ‘net show’ commands to validate the configuration

- Last, you will use ‘net rollback’ command to roll back to a previous configuration

Task 1: Retrieve system information

- Identify the switch image version:

# net show system

cumulus@spine3:mgmt:~$ net show system

Hostname......... spine3

Build............ Cumulus Linux 4.2.0

Uptime........... 1:18:20.020000

Model............ Cumulus VX

Memory........... 922MB

Vendor Name...... Cumulus Networks

Part Number...... 4.2.0

Base MAC Address. 44:38:39:00:00:20

Serial Number.... 44:38:39:00:00:20

Product Name..... VX

Task 2: Configure the switch with NCLU

- Display the switch ports:

cumulus@spine3:mgmt:~$ net show interface

State  Name  Spd  MTU    Mode      LLDP                    Summary

-----  ----  ---  -----  --------  ----------------------  --------------------------

UP     lo    N/A  65536  Loopback                          IP: 127.0.0.1/8

lo                                                  IP: ::1/128

UP     eth0  1G   1500   Mgmt      oob-mgmt-switch (swp4)  Master: mgmt(UP)

eth0                                                IP: 192.168.200.5/24(DHCP)

UP     mgmt  N/A  65536  VRF                               IP: 127.0.0.1/8

# net show interface

- Bring up the switch ports:

  - On the spine switches bring up swp1-4.

# net add interface swp1-4

cumulus@spine3:mgmt:~$ net add interface swp1-4

cumulus@spine3:mgmt:~$ net commit

cumulus@spine3:mgmt:~$ net show interface

State  Name  Spd  MTU    Mode      LLDP                    Summary

-----  ----  ---  -----  --------  ----------------------  --------------------------

UP     lo    N/A  65536  Loopback                          IP: 127.0.0.1/8

lo                                                  IP: ::1/128

UP     eth0  1G   1500   Mgmt      oob-mgmt-switch (swp4)  Master: mgmt(UP)

eth0                                                IP: 192.168.200.5/24(DHCP)

UP     swp1  1G   9216   Default

UP     swp2  1G   9216   Default

UP     swp3  1G   9216   Default

UP     swp4  1G   9216   Default

UP     mgmt  N/A  65536  VRF                               IP: 127.0.0.1/8

# net commit

  - On the leaf switches bring up ports swp1-2,5-6,8-9

# net add interface swp1-2,5-6,8-9

cumulus@leaf2:mgmt:~$ net add int swp1-2,5-6,8-9

cumulus@leaf2:mgmt:~$ net commit

cumulus@leaf2:mgmt:~$ net show interface

State  Name  Spd  MTU    Mode      LLDP                    Summary

-----  ----  ---  -----  --------  ----------------------  --------------------------

UP     lo    N/A  65536  Loopback                          IP: 127.0.0.1/8

lo                                                  IP: ::1/128

UP     eth0  1G   1500   Mgmt      oob-mgmt-switch (swp3)  Master: mgmt(UP)

eth0                                                IP: 192.168.200.4/24(DHCP)

UP     swp1  1G   9216   Default   spine4 (swp1)

UP     swp2  1G   9216   Default   spine3 (swp2)

UP     swp5  1G   9216   Default   leaf1 (swp5)

UP     swp6  1G   9216   Default   leaf1 (swp6)

UP     swp8  1G   9216   Default

UP     swp9  1G   9216   Default

UP     mgmt  N/A  65536  VRF                               IP: 127.0.0.1/8

# net commit

- On switch leaf1 add an alias to each of the interfaces which are part of the lab topology. The alias should describe the device which is connected to the interface. 
For example:

- Interface swp8 is connected to server host1

- Interface swp1 is connected to switch spine3

# net add interface <INTERFACE> alias <TEXT>

cumulus@leaf1:mgmt:~$ net add interface swp8 alias Connected to host1:Eth2

cumulus@leaf1:mgmt:~$ net add interface swp1 alias Connected to spine3:swp1

- View the changes in the commit buffer:

cumulus@leaf1:mgmt:~$ net pending

auto swp1

iface swp1

+    alias Connected to spine3:swp1

auto swp8

iface swp8

+    alias Connected to host1:Eth2

# net pending

- Commit the changes with a custom description:

cumulus@leaf1:mgmt:~$ net commit description Practice-1

auto swp1

iface swp1

+    alias Connected to spine3:swp1

auto swp8

iface swp8

+    alias Connected to host1:Eth2

net add/del commands since the last "net commit"

================================================

User     Timestamp                   Command

-------  --------------------------  -----------------------------------------------------

cumulus  2020-10-11 09:52:20.747134  net add interface swp8 alias Connected to host1:Eth2

cumulus  2020-10-11 09:52:30.206711  net add interface swp1 alias Connected to spine3:swp1

# net commit description <TEXT>

cumulus@leaf1:mgmt:~$ net show commit history

#  Date                 Description

-  -------------------  --------------------------------

3  2020-10-11 09:43:46  nclu "net commit" (user cumulus)

5  2020-10-11 09:57:55  nclu Practice-1

View the NCLU commit history:
# net show commit history

Task 3: Rollback the Configuration

- Rollback to the last commit:
# net rollback last

cumulus@leaf1:mgmt:~$ net rollback last

- Verify rollback was applied successfully:
# net show configuration

cumulus@leaf1:mgmt:~$ net show configuration

<output omitted>

interface swp1

interface swp8

Please note:

- At this point all configuration is deleted because the configuration has been reverted to a point before any configuration existed.

- Alternatively, you can roll back to any commit by referencing the unique commit number or description.

## PRACTICE 2: BASIC SWITCH FUNCTIONS

Practice Objectives:

In this practice session you will verify IP connectivity between servers in the lab.

- You will configure the servers IP settings – an IP address, a subnet mask and a default gateway.

- You will configure a bridge on each of the Cumulus Linux switches and add switch ports to the bridge.

- You will use ‘ping’ utility to verify communication between servers in your group.

- Last, you will observe how the switch forwarding database – the MAC address table – is built and maintained.

spine3

spine4

Topology Used in this Practice:

leaf2

leaf1

host3

host4

host2

host1

172.16.1.X

172.16.1.X

Eth2

172.16.1.X

swp1
trunk

swp1
trunk

swp2
trunk

Eth2

Eth2

Eth2

swp8
access

swp9
access

swp8
access

swp9
access

swp2
trunk

172.16.1.X

Task 1: Configure Servers IP Settings

- Access the servers and check interface ’eth2’ IP settings:

cumulus@host1:~$ ifconfig eth2

eth2: flags=4098<BROADCAST,MULTICAST>  mtu 1500

ether 44:38:39:00:00:11  txqueuelen 1000  (Ethernet)

RX packets 0  bytes 0 (0.0 B)

RX errors 0  dropped 0  overruns 0  frame 0

TX packets 0  bytes 0 (0.0 B)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0

# ifconfig <DEV>

- Configure an IP address and a subnet mask for interface ’eth2’.

Use the following table for IP address assignment. 
Use /24 as the subnet mask.

| Practice Lab Servers Properties |  |
| --- | --- |
| Server | ‘eth2’ IP Address |
| host1 | 172.16.1.1 |
| host2 | 172.16.1.2 |
| host3 | 172.16.1.3 |
| host4 | 172.16.1.4 |

# ifconfig <DEV> <IP/MASK>

cumulus@host1:~$ sudo ifconfig eth2 0.0.0.0

cumulus@host1:~$ sudo ifconfig eth2 172.16.1.1/24

cumulus@host1:~$ ifconfig eth2

eth2: flags=4098<BROADCAST,MULTICAST>  mtu 1500

ether 44:38:39:00:00:11  txqueuelen 1000  (Ethernet)

RX packets 0  bytes 0 (0.0 B)

RX errors 0  dropped 0  overruns 0  frame 0

TX packets 0  bytes 0 (0.0 B)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0        RX errors 0  dropped 3  overruns 0  frame 0

TX packets 10  bytes 1086 (1.0 KB)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0        RX errors 0  dropped 0  overruns 0  frame 0

TX packets 270015  bytes 51701450 (49.3 MiB)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0

- Verify a static route entry to 172.16.0.0/16 via interface ’eth2’:

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.1.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

# route

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2

If the static route is missing, add it:
# ip route add <ADDRESS/MASK> dev <DEV>

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      0.0.0.0         255.255.0.0     U     0      0        0 eth2

172.16.1.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

Task 2: Configure a Bridge

- Access the switches and reset configuration:

# net del all

cumulus@leaf1:mgmt:~$ net del all

cumulus@leaf1:mgmt:~$ net commit

# net commit

- On all four switches – leaves and spines - create a bridge add VLAN1  and set the inter switch links (swp1 and swp2) as trunk ports:

# net add bridge bridge ports <PORTS>

# net add bridge bridge vids <VLANS>

cumulus@leaf1:mgmt:~$ net add bridge bridge ports swp1-2

cumulus@leaf1:mgmt:~$ net add bridge bridge vids 1

- On the leaf switches only – leaf1 and leaf2 - set the host-facing ports, swp8 and swp9, as access ports in VLAN 1:

# net add interface <PORTS> bridge access <VLAN-ID>

cumulus@leaf1:mgmt:~$ net add bridge bridge ports swp8-9

cumulus@leaf1:mgmt:~$ net add interface swp8-9 bridge access 1

cumulus@leaf1:mgmt:~$ net commit

Commit changes:
# net commit

- Verify configuration:
# net show configuration

cumulus@leaf1:mgmt:~$  net show configuration

interface swp8

bridge-access 1

interface swp9

bridge-access 1

interface bridge

bridge-ports swp1 swp2 swp8 swp9

bridge-vids 1

bridge-vlan-aware yes

- Verify interfaces status:

cumulus@leaf1:mgmt:~$ net show interface

State  Name    Spd  MTU    Mode       LLDP                        Summary

-----  ------  ---  -----  ---------  --------------------------  --------------------------

UP     lo      N/A  65536  Loopback                               IP: 127.0.0.1/8

lo                                                         IP: ::1/128

UP     eth0    1G   1500   Mgmt       oob-mgmt-switch (swp2)      Master: mgmt(UP)

eth0                                                       IP: 192.168.200.3/24(DHCP)

UP     swp1    1G   9216   Access/L2  spine3 (swp1)               Master: bridge(UP)

UP     swp2    1G   9216   Access/L2  spine4 (swp2)               Master: bridge(UP)

UP     swp8    1G   9216   Access/L2  host1 (44:38:39:00:00:11)   Master: bridge(UP)

UP     swp9    1G   9216   Access/L2                              Master: bridge(UP)

UP     bridge  N/A  9216   Bridge/L2

UP     mgmt    N/A  65536  VRF                                    IP: 127.0.0.1/8

# net show interface

Please note:

Even though swp1-2 were configured in ‘trunk’ mode, they are displayed in the output in ‘Access’ mode. The reason is that currently only VLAN1 is configured and no frames are sent tagged over those ports. 
Once you configure additional VLANs, those ports will be displayed in ‘trunk’ mode indicating they are tagging frames.

Task 3: Observe a Switch’s Forwarding Database

cumulus@host1:~$ ifconfig eth2

eth2: flags=4163<UP,BROADCAST,RUNNING,MULTICAST>  mtu 1500

inet 172.16.1.1  netmask 255.255.255.0  broadcast 172.16.1.255

inet6 fe80::4638:39ff:fe00:11  prefixlen 64  scopeid 0x20<link>

ether 44:38:39:00:00:11  txqueuelen 1000  (Ethernet)

RX packets 1159  bytes 79499 (79.4 KB)

RX errors 0  dropped 182  overruns 0  frame 0

TX packets 71  bytes 13388 (13.3 KB)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0

Identify the MAC addresses of all four servers in your group.

Fill in the table with the MAC addresses of ’eth2’ interfaces of the servers:

| Server | MAC address of interface ’eth2’ |
| --- | --- |

cumulus@host1:~$ ping 172.16.1.3

PING 172.16.1.3 (172.16.1.3) 56(84) bytes of data.

64 bytes from 172.16.1.3: icmp_seq=1 ttl=64 time=2.99 ms

64 bytes from 172.16.1.3: icmp_seq=2 ttl=64 time=2.28 ms

Use ‘ping’ from one server to another server in the lab. 
For example, ping from server host1 to server host3.

cumulus@leaf2:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

1         bridge  swp1       44:38:39:00:00:09                                00:00:15

1         bridge  swp2       44:38:39:00:00:07                                00:00:01

1         bridge  swp2       44:38:39:00:00:11                                00:01:29

1         bridge  swp8       44:38:39:00:00:15                                00:00:23

Display the switch’s MAC address table. Identify on which switch ports the servers’ MAC addresses were learned?
# net show bridge macs

## PRACTICE 3: VLANs AND TRUNKING

Practice Objectives:

In this practice session you will configure and verify VLANs and trunking:

- You will configure two new VLANs and assign switch ports connected to servers to the configured VLANs.

- You will configure SVIs to allow inter-VLAN communication.

swp1
trunk

swp2
trunk

swp1
trunk

leaf1

spine4

spine3

leaf2

swp8
access

swp9
access

swp8
access

swp9
access

VLAN2

VLAN3

VLAN2

VLAN3

Topology Used in this Lab:

Task 1: Configuring VLANs and Trunking

- Access the switch that was assigned to you and reset configuration:

# net del all

cumulus@leaf1:mgmt:~$ net del all

cumulus@leaf1:mgmt:~$ net commit

# net commit

- On all four switches – spines and leaves - create a bridge and set the inter switch links, swp1 and swp2, as trunk ports:

# net add bridge bridge ports <PORTS>

cumulus@leaf1:mgmt:~$ net add bridge bridge ports swp1-2

On all four switches – spines and leaves -

cumulus@leaf1:mgmt:~$ net add bridge bridge vids 2,3

add VLANs 2-3 to the bridge:

- On leaf switches only leaf1 and leaf2 set the host-facing ports, swp8 and swp9, as access ports and associate them to the appropriate VLAN: 
interface swp8 in VLAN2 and interface swp9 in VLAN3
# net add interface <PORTS> bridge access <VLAN-ID>

cumulus@leaf1:mgmt:~$ net add bridge bridge ports swp8-9

cumulus@leaf1:mgmt:~$ net add interface swp8 bridge access 2

cumulus@leaf1:mgmt:~$ net add interface swp9 bridge access 3

cumulus@leaf1:mgmt:~$ net commit

Commit changes:

cumulus@leaf1:mgmt:~$ net show configuration

interface swp8

bridge-access 2

interface swp9

bridge-access 3

interface bridge

bridge-ports swp1 swp2 swp8 swp9

bridge-vids 1-3

bridge-vlan-aware yes

Verify configuration:

- Verify VLANs configuration:

# net show bridge vlan

cumulus@leaf1:mgmt:~$ net show bridge vlan

Interface  VLAN  Flags

---------  ----  ---------------------

swp1       1     PVID, Egress Untagged

2-3   []

swp2       1     PVID, Egress Untagged

2-3   []

swp8       2     PVID, Egress Untagged

swp9       3     PVID, Egress Untagged

Please note:

- Access ports are shown with a single line representing the VLAN associated to the port.

- Trunk ports are shown with multiple lines representing the VLANs associated with the trunk port.

Task 2: Servers’ IP settings

Access the servers and configure an IP address for interface ‘eth2’ according to the table below.

Servers in the same VLAN will be configured with IP addresses in the same subnet, hence they will be able to communicate over the layer 2 network.

| VLAN ID | Server | ‘eth2’ IP Address |
| --- | --- | --- |
| VLAN 2 | host1 | 172.16.2.18/24 |
| VLAN 3 | host2 | 172.16.3.19/24 |
| VLAN 2 | host3 | 172.16.2.28/24 |
| VLAN 3 | host4 | 172.16.3.29/24 |

- Configure the server’s IP address and subnet mask:

- Clear existing IP configuration:
# ifconfig eth2 0.0.0.0

- Configure an IP address and a subnet mask:

# ifconfig eth2 <IP/MASK>

- Verify IP configuration:

cumulus@host1:~$ sudo ifconfig eth2 0.0.0.0

cumulus@host1:~$ sudo ifconfig eth2 172.16.2.18/24

cumulus@host1:~$ ifconfig eth2

eth2: flags=4163<UP,BROADCAST,RUNNING,MULTICAST>  mtu 1500

inet 172.16.2.18  netmask 255.255.255.0  broadcast 172.16.2.255

inet6 fe80::4638:39ff:fe00:11  prefixlen 64  scopeid 0x20<link>

ether 44:38:39:00:00:11  txqueuelen 1000  (Ethernet)

RX packets 2946  bytes 202184 (202.1 KB)

RX errors 0  dropped 182  overruns 0  frame 0

TX packets 190  bytes 38098 (38.0 KB)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0

# ifconfig eth2

- Verify a static route entry to network 172.16.0.0/16 via the default gateway’s address. Use the default gateway address in the following table:

| VLAN ID | Default gateway address |
| --- | --- |
| VLAN 2 | 172.16.2.254/24 |
| VLAN 3 | 172.16.3.254/24 |

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

# ip route add <NET_ADDRESS/MASK> dev <DEV> via <IP>

[cumulus@host1 ~]# ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=4.11 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=2.58 ms

Use ‘ping’ to check communication between servers in the same VLAN. 
For example servers host1 and host3.

Task 3: Configuring SVIs for inter-VLAN routing

- On switch spine3 configure two SVIs (Switch VLAN Interfaces) that will be used for routing between VLAN2 and VLAN3:

  - Interface vlan2 will serve as the default gateway for VLAN2

  - Interface vlan3 will serve as the default gateway for VLAN3

Use the following table for IP address assignment:

| Interface vlan 2 | 172.16.2.254/24 |
| --- | --- |
| Interface vlan 3 | 172.16.3.254/24 |

cumulus@spine3:mgmt:~$  net add vlan 2 ip address 172.16.2.254/24

cumulus@spine3:mgmt:~$ net add vlan 3 ip address 172.16.3.254/24

cumulus@spine3:mgmt:~$ net commit

# net add vlan <VID> ip address <IP/MASK>

[cumulus@host1 ~]# ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=0.155 ms

64 bytes from 172.16.23.19: icmp_seq=2 ttl=63 time=0.192 ms64 bytes from 172.16.23.254: icmp_seq=2 ttl=64 time=0.362 ms

[cumulus@host1 ~]#  traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.340 ms  1.633 ms  1.607 ms

2  172.16.3.19 (172.16.3.19)  4.043 ms  4.023 ms  3.999 ms 2  172.16.3.19 (172.16.3.19)  0.155 ms  0.152 ms  0.137

Use ‘ping’ and ‘traceroute’ utilities to verify communication between hosts in different VLANs. For example, ping from server ‘host1’ in VLAN2 to server ‘host2’ in VLAN3.

## PRACTICE 4: CONFIGURING MLAG and VRR

Practice Objectives:

In this practice session you will configure the spine switches, spine3 and spine4, with MLAG and VRR towards the leaf switches.

- MLAG will make the spine switches to look and behave like a single Layer 2 switch towards the Layer 2 network.

- VRR will make the spine switches to look and behave like a single router providing default gateway redundancy.

- Interface ‘clag 1’ will aggregate swp1 on spine3 and swp2 on spine4 connected to switch leaf1. 
Switch leaf1 will be configured with a regular LAG.

- Interface ‘clag 2’ will aggregate swp2 on spine3 and swp1 on spine4 connected to switch leaf2. 
Switch leaf2 will be configured with a regular LAG.

swp1

swp1

swp2

Eth2

Eth2

Eth2

Eth2

access
VLAN2

swp2

172.16.2.X

172.16.3.X

172.16.2.X

172.16.3.X

swp8

swp9

access
VLAN3

swp8

swp9

access
VLAN3

access
VLAN2

swp1

swp2

swp1

swp2

clag id 1

clag id 1

VRRR

Topology used in this lab:

spine3

spine4

host1

host3

host2

host4

leaf1

leaf2

Task 1: LAG Configuration – Leaf Switches

- Access the leaf switches leaf1 and leaf2 and reset configuration:

# net del all

cumulus@leaf1:mgmt:~$ net del all

cumulus@leaf1:mgmt:~$ net commit

# net commit

- Create a bond named ‘BOND-TO-SPINES’, where bonds slaves are interfaces swp1 and swp2.

cumulus@leaf1:mgmt:~$ net add bond BOND-TO-SPINES bond slaves swp1-2

cumulus@leaf1:mgmt:~$ net commit

# net add bond <BOND-NAME> bond slaves <PORTS>

- Verify bonds configuration:

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND-TO-SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

Please note:

The bond interface is down because its peer (the MLAG peer) was not configured yet.

- Create a bridge and add the bond interface, BOND-TO-SPINES, and the host facing interfaces, swp8 and swp9, to the bridge:

cumulus@leaf1:mgmt:~$ net add bridge bridge ports swp8-9,BOND-TO-SPINES

- Configure VLANs 2-3 and associate swp8 to VLAN2 and swp9 to VLAN3:

cumulus@leaf1:mgmt:~$ net add interface swp8 bridge access 2

cumulus@leaf1:mgmt:~$ net add interface swp9 bridge access 3

cumulus@leaf1:mgmt:~$ net commit

Please note:

Commands are demonstrated on switch leaf1. 
Switch leaf2 should be configured similarly.

Task 2: Configuring MLAG – spine switches

- Access the switches and reset the configuration:

# net del all

cumulus@spine3:mgmt:~$ net del all

cumulus@spine3:mgmt:~$ net commit

# net commit

cumulus@spine4:mgmt:~$ net show interface eth0

Name  MAC                Speed  MTU   Mode

--  ----  -----------------  -----  ----  ----

UP  eth0  44:38:39:00:00:22  1G     1500  Mgmt

IP Details

-------------------------  ----------------

IP:                        192.168.200.6/24

cumulus@spine3:mgmt:~$ net show interface eth0

Name  MAC                Speed  MTU   Mode

--  ----  -----------------  -----  ----  ----

UP  eth0  44:38:39:00:00:20  1G     1500  Mgmt

IP Details

-------------------------  ----------------

IP:                        192.168.200.5/24

Write down, on a side note, the IP addresses of the management ports, eth0. Those IP addresses will be configured as the MLAG backup-IPs.

- Configure the spine switches, spine3 and spine4, as MLAG peers.

# net add clag peer sys-mac <MAC> interface <PEERLINK-INTERFACES> <ROLE> backup-ip <IP>

Please note:

- Interfaces swp3 and swp4 will be configured as the MLAG peerlink.

- Switch spine3 is configured as the MLAG primary and spine4 as the secondary.

- On switch spine3 use spine4’s IP as the backup-IP and vice-versa.

cumulus@spine3:mgmt:~$ net add clag peer	sys-mac 44:38:39:FF:00:01 interface swp3-4 primary backup-ip 192.168.200.6 vrf mgmt

cumulus@spine4:mgmt:~$ net add clag peer	sys-mac 44:38:39:FF:00:01 interface swp3-4 secondary backup-ip 192.168.200.5 vrf mgmt

- Configure bridge settings – VLANs and STP priority:

cumulus@spine3:mgmt:~$ net add vlan 2-3

cumulus@spine3:mgmt:~$ net add bridge stp treeprio 4096

cumulus@spine4:mgmt:~$ net add vlan 2-3

cumulus@spine4:mgmt:~$ net add bridge stp treeprio 4096

- Commit the changes:

cumulus@spine3:mgmt:~$ net commit

cumulus@spine4:mgmt:~$ net commit

Task 3: Configuring CLAG interfaces - spine switches

- Configure two CLAG interfaces on each of the spine switches.

- Interface ‘clag 1’ will aggregate swp1 on spine3 and swp2 on spine4 which are connected to switch leaf1

swp2

swp1

leaf1

spine4

spine3

clag id 1

swp2

swp1

cumulus@spine4:mgmt:~$ net add clag port bond LEAF1 interface swp2 clag-id 1

cumulus@spine3:mgmt:~$ net add clag port bond LEAF1 interface swp1 clag-id 1

# net add clag port bond <BONDNAME> interface <PORTS> clag-id <ID>

- Interface ‘clag 2’ will aggregate swp2 on spine3 and swp1 on spine4 which are connected to switch leaf2.

swp1

swp2

swp2

swp1

swp2

clag id 1

spine4

spine3

leaf2

cumulus@spine4:mgmt:~$ net add clag port bond LEAF2 interface swp1 clag-id 2

cumulus@spine3:mgmt:~$ net add clag port bond LEAF2 interface swp2 clag-id 2

- Commit the changes:

cumulus@spine3:mgmt:~$ net commit

cumulus@spine4:mgmt:~$ net commit

- View MLAG resulting configuration:

cumulus@spine3:mgmt:~$ net show configuration

interface LEAF1

bond-slaves swp1

clag-id 1

interface LEAF2

bond-slaves swp2

clag-id 2

interface bridge

bridge-ports peerlink LEAF1 LEAF2

bridge-vids 2-3

bridge-vlan-aware yes

mstpctl-treeprio 4096

interface peerlink

bond-slaves swp3 swp4

interface peerlink.4094

clagd-backup-ip 192.168.200.6 vrf mgmt

clagd-peer-ip linklocal

clagd-priority 1000

clagd-sys-mac 44:38:39:FF:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is alive

Our Priority, ID, and Role:  1000 44:38:39:00:00:20  primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:22  secondary

Peer Interface and IP: peerlink.4094 linklocal

Backup IP: 192.168.200.6 (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

Verify that MLAG protocol is up: 
# net show clag

Task 4: Servers’ IP settings

| VLAN ID | Server | ‘eth2’ IP Address |
| --- | --- | --- |
| VLAN 2 | host1 | 172.16.2.18/24 |
| VLAN 3 | host2 | 172.16.3.19/24 |
| VLAN 2 | host3 | 172.16.2.28/24 |
| VLAN 3 | host4 | 172.16.3.29/24 |

- Access the servers and configure an IP address for interface ‘eth2’ (see tables below).

Configure the server’s IP address and subnet mask:

- Clear existing IP configuration:
# ifconfig eth2 0.0.0.0

- Configure an IP address and a subnet mask:

# ifconfig eth2 <IP/MASK>

- Verify IP configuration:

cumulus@host1:~$ sudo ifconfig eth2 0.0.0.0

cumulus@host1:~$ sudo ifconfig eth2 172.16.2.18/24

cumulus@host1:~$ ifconfig eth2

eth2: flags=4163<UP,BROADCAST,RUNNING,MULTICAST>  mtu 1500

inet 172.16.2.18  netmask 255.255.255.0  broadcast 172.16.2.255

inet6 fe80::4638:39ff:fe00:11  prefixlen 64  scopeid 0x20<link>

ether 44:38:39:00:00:11  txqueuelen 1000  (Ethernet)

RX packets 3  bytes 180 (180.0 B)

# ifconfig eth2

- Verify a static route entry to network 172.16.0.0/16 via the default gateway’s address.

Use the following table for default gateway assignment.

| VLAN ID | Default gateway address |
| --- | --- |
| vlan 2 | 172.16.2.254/24 |
| vlan 3 | 172.16.3.254/24 |

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

# ip route add <NET_ADDRESS/MASK> dev <DEV> via <IP>

[cumulus@host1 ~]# ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

Use ‘ping’ to check communication between servers in the same VLAN. 
For example, server host1 and host3 in VLAN2.

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

Verify that the MLAG switches have their MAC address tables synchronized.

Please note:

- Once the MLAG configuration is completed, the spine switches appear as a single 
Layer 2 switch towards to Layer 2 network. Hence, they are capable to provide an efficient load balancing and better network utilization for the layer 2 network.

- In the following task additionally, the spine switches will be configured as VRR routers. Thus, they will provide default gateway redundancy and efficient load balancing towards the layer 3 network.

Task 5: Configuring VRR

Topology used in this task:

leaf2

leaf1

spine4

spine3

Vlan 3 VIP: 172.16.3.254

Vlan 2 VIP: 172.16.2.254

swp1

swp1

swp2

Eth2

Eth2

Eth2

Eth2

access
VLAN2

swp2

172.16.2.X

172.16.3.X

172.16.2.X

172.16.3.X

swp8

swp9

access
VLAN3

swp8

swp9

access
VLAN3

access
VLAN2

swp1

swp2

swp1

swp2

clag id 1

clag id 1

swp4

swp3

- Configure two SVIs (Switch Virtual Interfaces) on each of the spine switches, interface vlan 2 and interface vlan 3.

  - Configure an IP for each of the vlan interfaces.

  - Configure a VIP (Virtual IP) and a VMAC (Virtual MAC).

  - Use the following table for address assignment. Use /24 as the subnet mask.

| SWITCH SPINE3 |  |  |  |
| --- | --- | --- | --- |
| VLAN | SVI | VIP | VMAC |
| vlan 2 | 172.16.2.252/24 | 172.16.2.254/24 | 00:00:5e:00:01:02 |
| vlan 3 | 172.16.3.252/24 | 172.16.3.254/24 | 00:00:5e:00:01:03 |
| SWITCH SPINE4 |  |  |  |
| VLAN | SVI | VIP | VMAC |
| vlan 2 | 172.16.2.253/24 | 172.16.2.254/24 | 00:00:5e:00:01:02 |
| vlan 3 | 172.16.3.253/24 | 172.16.3.254/24 | 00:00:5e:00:01:03 |

# net add vlan <VLAN_ID> ip address <IP/MASK>

cumulus@spine4:mgmt:~$ net add vlan 2 ip address 172.16.2.253/24

cumulus@spine4:mgmt net add vlan 2 ip address-virtual 00:00:5e:00:01:02 172.16.2.254/24

cumulus@spine4:mgmt net add vlan 3 ip address 172.16.3.253/24

cumulus@spine4:mgmt net add vlan 3 ip address-virtual 00:00:5e:00:01:03 172.16.3.254/24

cumulus@spine4:~$ net commit

cumulus@spine3:mgmt:~$ net add vlan 2 ip address 172.16.2.252/24

cumulus@spine3:mgmt:~$ net add vlan 2 ip address-virtual 00:00:5e:00:01:02 172.16.2.254/24

cumulus@spine3:mgmt:~$ net add vlan 3 ip address 172.16.3.252/24

cumulus@spine3:mgmt:~$ add vlan 3 ip address-virtual 00:00:5e:00:01:03 172.16.3.254/24

cumulus@spine3:mgmt:~$ commit

# net add vlan <VLAN_ID> ip address-virtual <VMAC> <VIP/MASK>

cumulus@spine3:mgmt:~$ net show interface vlan2-v0

Name      MAC                Speed  MTU   Mode

--  --------  -----------------  -----  ----  ------------

UP  vlan2-v0  00:00:5e:00:01:02  N/A    9216  Interface/L3

IP Details

-------------------------  ---------------

IP:                        172.16.2.254/24

cumulus@spine3:mgmt:~$ net show interface

State  Name           Spd   MTU    Mode          LLDP         Summary

-----  -------------  ----  -----  ------------  -----------  -------------------

UP     vlan2          N/A  9216   Interface/L3                IP: 172.16.2.252/24

UP     vlan2-v0       N/A  9216   Interface/L3                IP: 172.16.2.254/24

UP     vlan3          N/A  9216   Interface/L3                IP: 172.16.3.252/24

UP     vlan3-v0       N/A  9216   Interface/L3                IP: 172.16.3.254/24

Verify VRR configuration:

[cumulus@host1 ~]# ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]# traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

Verify VRR operation.
Use ‘ping’ and ‘traceroute’ utilities to verify communication between hosts in different VLANs. 
For example, from ‘host1’ in VLAN 2 to ‘host2’ in VLAN 3.

- Verify MLAG/VRR failover.
Use continuous ‘ping’ between hosts in different VLANs. 
For example, from server ‘host1’ in VLAN 2 to server ‘host2’ in VLAN 3.

While ‘ping’ is running, reboot switch spine3. Was the traffic disrupted?

After switch spine3 reboots, reboot switch spine4. Was the traffic disputed now?

What are your conclusions regarding MLAG/VRR failover?

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

Practice Objectives:

In this practice session you will configure BGP Unnumbered:

- Spine switches will be configured in the same AS and each of the leaf switches will be configured in its own AS.

- BGP unnumbered will be configured on all four switches.

- eBGP sessions will be established between the spine and leaf switches.

- Leaf switches will advertise their local IP prefixes.

- By the end of this practice session you will achieve end-to-end connectivity between all servers in your group, over the BGP Autonomous Systems.

Please note:

- Commands are demonstrated on switches leaf1 and spine3. You should apply similar commands on the other two switches in your group.

Topology used in this practice session:

swp2

swp2

swp8

swp9

swp1

swp2

swp2

swp1

swp8

swp9

swp1

swp1

AS 65101

AS 65102

AS 65100

spine4

spine3

leaf2

leaf1

Task 1: Starting FRR

- Access the switches and reset the configuration:

# net del all

cumulus@leaf1:mgmt:~$ net del all

cumulus@leaf1:mgmt:~$ net commit

# net commit

cumulus@leaf1:mgmt:~$ sudo vi /etc/frr/daemons

zebra=yes

bgpd=yes

Start FRR routing daemons. 
Edit the /etc/frr/daemons file. Set to ‘yes’ both zebra and bgpd.
Save and exit.

- Restart FRR Service.

cumulus@leaf1:mgmt:~$ sudo systemctl restart frr

Task 2: Configuring loopback interfaces

| Switch | Loopback IP address |
| --- | --- |
| leaf1 | 172.16.100.1/32 |
| leaf2 | 172.16.100.2/32 |
| spine3 | 172.16.100.3/32 |
| spine4 | 172.16.100.4/32 |

- On each of the switches configure a loopback interface.

Use the following table for IP address assignment:

cumulus@leaf1:mgmt:~$ net add loopback lo ip address 172.16.100.1/32

# net add loopback lo ip address <IP/MASK>

cumulus@spine3:mgmt:~$ net add loopback lo ip address 172.16.100.3/32

Task 3: Configuring BGP Unnumbered

- Configure BGP unnumbered:

# net add bgp autonomous-system <LOCAL-AS>

cumulus@spine3:mgmt:~$ net add bgp autonomous-system 65100

cumulus@spine3:mgmt:~$ net add bgp neighbor swp1-2 interface remote-as external

cumulus@leaf1:mgmt:~$ net add bgp autonomous-system 65101

cumulus@leaf1:mgmt:~$ net add bgp neighbor swp1-2 interface remote-as external

# net add bgp neighbor <INTERFACE> interface remote-as external

- Advertise the loopback addresses and commit the changes.

# net add bgp network <IP/MASK>

cumulus@leaf1:mgmt:~$ net add bgp network 172.16.100.1/32

cumulus@leaf1:mgmt:~$ net commit

cumulus@spine3:mgmt:~$ net add bgp network 172.16.100.3/32

cumulus@spine3:mgmt:~$ net commit

Please note:

Commands are demonstrated on switches leaf1 and spine3. You should apply similar commands on the other two switches in your group.

- Verify configuration:

cumulus@leaf1:mgmt:~$ net show configuration

router bgp 65101

bgp router-id 172.16.100.1

neighbor swp1 interface remote-as external

neighbor swp2 interface remote-as external

address-family ipv4 unicast

network 172.16.100.1/32

interface lo

address 172.13.100.1/32

- Verify eBGP sessions were established: 
Each switch should be able to see two eBGP neighbors, via interfaces swp1 and wp2.

cumulus@leaf1:mgmt:~$ net show bgp summary

show bgp ipv4 unicast summary

=============================

BGP router identifier 172.16.100.1, local AS number 65101 vrf-id 0

BGP table version 3

RIB entries 7, using 1344 bytes of memory

Peers 2, using 43 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd

spine3(swp1)    4      65100      1295      1294        0    0    0 00:58:16            2

spine4(swp2)    4      65100       349       420        0    0    0 00:15:22            2

Total number of neighbors 2

# net show bgp summary

- Verify loopback prefixes are reachable:

- Each switch should have in its routing table the prefixes of the other switch’s loopback interfaces.

cumulus@leaf1:mgmt:~$ net show route bgp

RIB entry for bgp

=================

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued route, r - rejected route

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:06:41

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:06:41

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:05:52

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:05:34

# net show route bgp

Please note:

- NEXT-HOP is the neighbor’s IPv6 Link Local Address

- BGP multipath is enabled

Task 4: Advertise local IP prefixes

- On the leaf switches, assign IP addresses to the host-facing interfaces,

swp8 and swp9.
Advertise the IP prefixes in the BGP process.
Use the IP addresses listed in the below topology. Use /24 for the subnet mask.

swp8

swp1

swp8

swp9

host1

172.16.18.2

host2

172.16.19.2

host3

172.16.28.2

host4

172.16.29.2

172.16.18.1

172.16.19.1

swp9

172.16.28.1

172.16.29.1

loopback

172.16.100.1

172.16.100.2

loopback

172.16.100.4

loopback

loopback

172.16.100.3

swp2

swp2

swp1

spine4

spine3

leaf2

leaf1

# net add interface <INTERFACE> ip add <IP/MASK>

# net add bgp network <IP/MASK>

cumulus@leaf1:mgmt:~$ net add interface swp8 ip add 172.16.18.1/24

cumulus@leaf1:mgmt:~$ net add interface swp9 ip add 172.16.19.1/24

cumulus@leaf1:mgmt:~$ net add bgp network 172.16.18.0/24

cumulus@leaf1:mgmt:~$ net add bgp network 172.16.19.0/24

cumulus@leaf1:mgmt:~$ net commit

cumulus@leaf2:mgmt:~$ net add interface swp8 ip add 172.16.28.1/24

cumulus@leaf2:mgmt:~$ net add interface swp9 ip add 172.16.29.1/24

cumulus@leaf2:mgmt:~$ net add bgp network 172.16.28.0/24

cumulus@leaf2:mgmt:~$ net add bgp network 172.16.29.0/24

cumulus@leaf2:mgmt:~$ net commit

- Verify that the remote leaf switch has learned the IP prefixes:

- Switch leaf1 should have in its routing table the IP prefixes advertise by switch leaf2:

  - 172.16.28.1/24

  - 172.16.29.0/24

- Switch leaf2 should have in its routing table the IP prefixes advertise by switch leaf1:

  - 172.16.18.1/24

  - 172.16.19.0/24

# net show route bgp

cumulus@leaf1:mgmt:~$ net show route bgp

RIB entry for bgp

=================

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued route, r - rejected route

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:08

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:08

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:08

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:08

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:21:36

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:21:36

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:20:47

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:20:29

Task 5: Verify end-to-end connectivity

- Configure the servers IP settings. Configure an IP address and a subnet mask for interface ’eth2’. Use the following tables for IP address assignment. 
Use /24 as the subnet mask.

| Server | ’eth2’ IP Address | Next Hop to 172.16.0.0/16 |
| --- | --- | --- |
| host1 | 172.16.18.2 | 172.16.18.1 |
| host2 | 172.16.19.2 | 172.16.19.1 |
| host3 | 172.16.28.2 | 172.16.28.1 |
| host4 | 172.16.29.2 | 172.16.29.1 |

- Clear existing IP configuration:
# ifconfig <DEV> 0.0.0.0

- Configure an IP address and a subnet mask:
# ifconfig <DEV> <IP/MASK>

cumulus@host1:~$ sudo ifconfig eth2 0.0.0.0

cumulus@host1:~$ sudo ifconfig eth2 172.16.18.2/24

cumulus@host1:~$ ifconfig eth2

eth2: flags=4163<UP,BROADCAST,RUNNING,MULTICAST>  mtu 1500

inet 172.16.18.2  netmask 255.255.255.0  broadcast 172.16.18.255

inet6 fe80::4638:39ff:fe00:11  prefixlen 64  scopeid 0x20<link>

ether 44:38:39:00:00:11  txqueuelen 1000  (Ethernet)

RX packets 3  bytes 180 (180.0 B)

RX errors 0  dropped 0  overruns 0  frame 0

TX packets 270015  bytes 51701450 (49.3 MiB)

TX errors 0  dropped 0 overruns 0  carrier 0  collisions 0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

Add a route entry to 172.16.0.0/16 via interface ’eth2’:
# ip route add <ADDRESS/MASK> dev <DEV> via <IP_ADDRESS>

- Use ‘ping’ and ‘traceroute’ utilities to verify communication between servers in different Autonomous Systems. 

For example, ping from server ‘host1’ (AS 65101) to server ‘host3’ (in AS 65102).

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

Please note:

The next practice relies on BGP Unnumbered configuration. Please make sure to save the configuration before exiting.

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

Practice Objectives:

In this practice session you will configure VXLAN with EVPN:

- The leaf switches will be configured as the VTEPs

- EVPN will be configured as the VXLAN control plane

- Two VXLAN Network IDs (VNIs) will be configured:

  - VNI-10 will connect VLAN 10 on switch leaf1 and VLAN 100 on switch leaf2.

  - VNI-20 will connect VLAN 20 on switch leaf1 and VLAN 200 on switch leaf2.

- Hosts in each VNI will be able to communicate in layer 2 over the underlay layer 3 network.

Please note:

The configuration in this practice session relies on the previous practice. 
Make sure that BGP Unnumbered is properly configured and fully operational.

swp1

Swp8

Swp9

Swp8

Swp9

swp2

swp1

VLAN10

VLAN20

VLAN100

VLAN200

172.16.10.18/24

172.16.20.19/24

172.16.10.28/24

172.16.20.29/24

swp2

swp2

swp1

swp2

swp1

VNI-10

VNI-20

Topology used in this practice session:

spine4

spine3

leaf2

leaf1

host4

host3

host2

host1

Task 1: Configuring servers IP settings

- Configure an IP address and a subnet mask for interface ’eth2’. 
Use the following tables for IP address assignment. Use /24 as the subnet mask.

| Server | ETH2 IP Address |
| --- | --- |
| host1 | 172.16.10.18 |
| host2 | 172.16.20.19 |
| host3 | 172.16.10.28 |
| host4 | 172.16.20.29 |

- Clear existing IP configuration:
# ifconfig <DEV> 0.0.0.0

- Configure an IP address and a subnet mask:
# ifconfig <DEV< <IP/MASK>

Task 2: Configuring VLANs

- Access the leaf switches and configure the host-facing ports swp8 and swp9. 
First clear any existing configuration, then assign the ports to the appropriate VLAN:

  - Switch leaf1 - assign swp8 to VLAN 10 and swp9 to VLAN 20

  - Switch leaf2 - assign swp8 to VLAN 100 and swp9 to VLAN 200

# net add interface <INTERFACE> bridge access <VLAN>

cumulus@leaf1:mgmt:~$ net del interface swp8-9

cumulus@leaf1:mgmt:~$ net add interface swp8 bridge access 10

cumulus@leaf1:mgmt:~$ net add interface swp9 bridge access 20

cumulus@leaf1:mgmt:~$ net commit

cumulus@leaf1:mgmt:~$ net del interface swp8-9

cumulus@leaf2:mgmt:~$ net add interface swp8 bridge access 100

cumulus@leaf2:mgmt:~$ net add interface swp9 bridge access 200

cumulus@leaf2:mgmt:~$ net commit

Task 3: Configuring VNIs

- Access the leaf switches and configure the VNIs:

  - Create a VXLAN interface with a unique ID

  - Map the VXLAN to a VLAN

  - Configure the VXLAN’s local tunnel IP (use the VTEPs loopback)

# net add vxlan <VXLAN-NAME> vxlan id <VXLAN-ID>

# net add vxlan <VXLAN-NAME> bridge access <VLAN-ID>

# net add vxlan <VXLAN-NAME> vxlan local-tunnelip <IP-ADRESS>

# net commit

- On switch leaf1: 
VNI-10 will be mapped to VLAN 10 and VNI-20 will be mapped to VLAN 20.
VXLAN’s local tunnel IP is 172.16.100.1

- On switch leaf2: 
VNI-10 will be mapped to VLAN 100 and VNI-20 will be mapped to VLAN 200.
VXLAN’s local tunnel IP is 172.16.100.2

cumulus@leaf2:mgmt:~$ net add vxlan VNI-10 vxlan id 10

cumulus@leaf2:mgmt:~$ net add vxlan VNI-10 bridge access 100

cumulus@leaf2:mgmt:~$ net add vxlan VNI-10 vxlan local-tunnelip 172.16.100.2

cumulus@leaf2:mgmt:~$ net add vxlan VNI-20 vxlan id 20

cumulus@leaf2:mgmt:~$ net add vxlan VNI-20 bridge access 200

cumulus@leaf2:mgmt:~$ net add vxlan VNI-20 vxlan local-tunnelip 172.16.100.2

cumulus@leaf2:mgmt:~$ net commit

cumulus@leaf1:mgmt:~$ net add vxlan VNI-10 vxlan id 10

cumulus@leaf1:mgmt:~$ net add vxlan VNI-10 bridge access 10

cumulus@leaf1:mgmt:~$ net add vxlan VNI-10 vxlan local-tunnelip 172.16.100.1

cumulus@leaf1:mgmt:~$ net add vxlan VNI-20 vxlan id 20

cumulus@leaf1:mgmt:~$ net add vxlan VNI-20 bridge access 20

cumulus@leaf1:mgmt:~$ net add vxlan VNI-20 vxlan local-tunnelip 172.16.100.1

cumulus@leaf1:mgmt:~$ net commit

Task 4: Configuring EVPN

- Access the spine switches and configure EVPN.

Switch spine4 should be configured similarly.

cumulus@spine3:mgmt:~$ net add bgp evpn neighbor swp1-2 activate

cumulus@spine3:mgmt:~$ net commit

# net add bgp evpn neighbor <INTERFACE> activate

- Access the leaf switches and configure EVPN to advertise all VNIs (EVPN allows VTEPs to exchange VNI membership information).
Switch leaf2 should be configured similarly.

# net add bgp evpn neighbor <INTERFACE> activate

cumulus@leaf1:mgmt:~$ net add bgp evpn neighbor swp1-2 activate

cumulus@leaf1:mgmt:~$ net add bgp evpn advertise-all-vni

cumulus@leaf1:mgmt:~$ net commit

cumulus@leaf1:~$ net commit

# net add bgp evpn advertise-all-vni

- Verify VXLAN Information.

# net show evpn vni <VNI>

cumulus@leaf1:mgmt:~$ net show evpn vni 10

VNI: 10

Type: L2

Tenant VRF: default

VxLAN interface: VNI-10

VxLAN ifIndex: 12

Local VTEP IP: 172.16.100.1

Mcast group: 0.0.0.0

Remote VTEPs for this VNI:

172.16.100.2 flood: HER

Number of MACs (local and remote) known for this VNI: 2

Number of ARPs (IPv4 and IPv6, local and remote) known for this VNI: 0

Advertise-gw-macip: No

Task 5: Verify end-to-end communication

- Each VNI is a logical layer 2 network over the underlay layer 3 network.
Logically there are no layer 3 hops between hosts in the same VNI.

- Use ‘ping’ and ‘traceroute’ utilities to verify end-to-end communication between hosts in a VNI. For example, from host ‘host1’ to ‘host3’ that are configured in VNI-10.

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

Please note: 
The next practice relies on VXLAN with EVPN configuration. Please make sure to save the configuration before exiting.

## PRACTICE 7: DISTRIBUTED ASYMMETRIC VXLAN ROUTING

Practice Objectives:

In this practice session you will configure distributed asymmetric VXLAN routing.

- VTEPs (leaf switches) will be configured as the distributed anycast default gateway, i.e., each VTEP will be configured with a VIP and VMAC for each of the subnets.
Each VTEP will serve as the default gateway for its locally connected hosts.

- In asymmetric VXLAN routing only the ingress VTEP perform the routing, while the egress VTEP perform VXLAN bridging only.

Please note:

The configuration in this practice session relies on the previous practice. Make sure that BGP Unnumbered and VXLAN with EVPN are properly configured and fully operational.

Topology used in this practice session:

spine4

spine3

swp1

swp8

swp9

swp8

swp9

swp2

swp1

VLAN10

VLAN20

VLAN100

VLAN200

swp2

swp2

swp1

swp2

swp1

VNI-10

VNI-20

172.16.10.18/24

Default-gateway:
172.16.10.254

172.16.20.19/24

Default-gateway:
172.16.20.254

172.16.10.28/24

Default-gateway:
172.16.10.254

172.16.20.29/24

Default-gateway:
172.16.20.254

BGP Unnumbered

leaf2

leaf1

host4

host3

host2

host1

Task 1: Configuring servers IP settings

- Add a static route entry to network 172.16.0.0/16 via the default gateway’s IP:

- Servers ‘host1’ and ‘host3’ are in VNI-10, their default gateway is 172.16.10.254

- Servers ‘host2’ and ‘host4’ are in VNI-20, their default gateway is 172.16.20.254

| Server | Default gateway |
| --- | --- |
| host1 | 172.16.10.254 |
| host2 | 172.16.20.254 |
| host3 | 172.16.10.254 |
| host4 | 172.16.20.254 |

[cumulus@host1 ~]# sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.10.254

# ip route add <NET_ADD/MASK> dev <DEV> via <IP>

Task 2: Configuring SVIs on the VTEPs

- Access the leaf switch and configure the SVIs and the anycast default gateway’s VMAC and VIP. 
Assign IP addresses according the following tables. Use /24 as the subnet mask.

Switch leaf1:

| SVI | IP | Default gateway’s VIP | Default gateway’s VMAC |
| --- | --- | --- | --- |
| VLAN 10 | 172.16.10.252 | 172.16.10.254 | 00:00:5e:00:01:01 |
| VLAN 20 | 172.16.20.252 | 172.16.20.254 | 00:00:5e:00:01:02 |

Switch leaf2:

| SVI | IP | Default gateway’s VIP | Default gateway’s VMAC |
| --- | --- | --- | --- |
| VLAN 100 | 172.16.10.253 | 172.16.10.254 | 00:00:5e:00:01:01 |
| VLAN 200 | 172.16.20.253 | 172.16.20.254 | 00:00:5e:00:01:02 |

# net add vlan <VLAN> ip address <IP>

cumulus@leaf2:mgmt:~$ net add vlan 100 ip address 172.16.10.253/24

cumulus@leaf2:mgmt:~$ net add vlan 100 ip address-virtual 00:00:5e:00:01:01 172.16.10.254/24

cumulus@leaf2:mgmt:~$ net add vlan 200 ip address 172.16.20.253/24

cumulus@leaf2:mgmt:~$ net add vlan 200 ip address-virtual 00:00:5e:00:01:02 172.16.20.254/24

cumulus@leaf2:mgmt:~$ net commit

cumulus@leaf1:mgmt:~$ net add vlan 10 ip address 172.16.10.252/24

cumulus@leaf1:mgmt:~$ net add vlan 10 ip address-virtual 00:00:5e:00:01:01 172.16.10.254/24

cumulus@leaf1:mgmt:~$ net add vlan 20 ip address 172.16.20.252/24

cumulus@leaf1:mgmt:~$ net add vlan 20 ip address-virtual 00:00:5e:00:01:02 172.16.20.254/24

cumulus@leaf1:mgmt:~$ net commit

# net add vlan <VLAN> ip address-virtual <VMAC> <VIP/MASK>

Please note:

The VMAC and VIP must be identical on both VTEPs in order to implement anycast default gateway and to allow proper inter-VXLAN routing.

Task 3: Test inter-VXLAN communication

- Use ‘ping’ and ‘traceroute’ utilities to verify end-to-end communication between hosts in different VNIs. 
For example, ping from host ‘host1’ in VNI-10 to ‘host2’ in VNI-20. 
In this case the egress switch leaf1 will perform VXLAN routing between the source and destination VNIs, while the egress switch leaf2 will bridge between destination VNI and the destination VLAN.

[cumulus@host1 ~]# ping 172.16.20.19

PING 172.16.20.19 (172.16.20.19) 56(84) bytes of data.

64 bytes from 172.16.20.19: icmp_seq=1 ttl=63 time=3.48 ms

64 bytes from 172.16.20.19: icmp_seq=2 ttl=63 time=1.15 ms

[cumulus@host1 ~]# traceroute 172.16.20.19

traceroute to 172.16.20.19 (172.16.20.19), 30 hops max, 60 byte packets

1  172.16.10.254 (172.16.10.254)  1.271 ms  1.431 ms  1.412 ms

2  172.16.20.19 (172.16.20.19)  1.564 ms  1.593 ms  1.595 ms

## PRACTICE 8: DISTRIBUTED SYMMETRIC VXLAN ROUTING

Practice Objectives:

In this practice session you will configure distributed symmetric VXLAN routing.

- VTEPs (leaf switches) will be configured as the distributed anycast default gateway, i.e., each VTEP will be configured with a VIP and VMAC for each of the subnets.
Each VTEP will serve as the default gateway for its locally connected hosts.

- In symmetric VXLAN routing both the ingress and egress VTEPs perform the routing.

- A new special transit VNI is used for all routed VXLAN traffic, called the L3-VNI.

Please note:

The configuration in this practice session relies on the previous practice. Make sure that BGP Unnumbered and VXLAN with EVPN are properly configured and fully operational. Test and verify that hosts in the same VNI can communicate with each other.

Topology used in this practice session:

spine4

spine3

swp1

swp8

swp9

swp8

swp9

swp2

swp1

VLAN10

VLAN20

VLAN100

VLAN200

swp2

swp2

swp1

swp2

swp1

VNI-10

VNI-20

172.16.10.18/24

Default-gateway:
172.16.10.254

172.16.20.19/24

Default-gateway:
172.16.20.254

172.16.10.28/24

Default-gateway:
172.16.10.254

172.16.20.29/24

Default-gateway:
172.16.20.254

BGP Unnumbered

leaf2

leaf1

host4

host3

host2

host1

Task 1: Configuring the L3-VNI

- Access both leaf switches and configure a per-tenant VXLAN interface.
Use VNI 4001.

cumulus@leaf1:mgmt:~$ net add vlan 4001

cumulus@leaf1:mgmt:~$ net add vxlan VNI-4001 vxlan id 4001

cumulus@leaf1:mgmt:~$ net add vxlan VNI-4001 bridge access 4001

cumulus@leaf1:mgmt:~$ net add vxlan VNI-4001 vxlan local-tunnelip 172.16.100.1

cumulus@leaf1:mgmt:~$ net add vrf VRFA

cumulus@leaf1:mgmt:~$ net add vlan 4001 vrf VRFA

Access both leaf switches and configure an SVI for the L3-VNI.

cumulus@leaf1:mgmt:~$ net add vrf VRFA vni 4001

cumulus@leaf1:mgmt:~$ net commit

Access both leaf switches and configure the VRF to L3-VNI mapping.

Please note:

Switch leaf2 should be configured similarly.

Task 2: Configuring Anycast Gateways

- Access the leaf switches and configure the SVIs and associate them to the L3-VNI Configure the anycast default gateway’s VMAC and VIP. 
Assign IP addresses according the following tables. Use /24 as the subnet mask.

| SVI | IP |
| --- | --- |
| VLAN 10 | 172.16.10.252 |
| VLAN 20 | 172.16.20.252 |

Switch leaf1:

| SVI | IP |
| --- | --- |
| VLAN 100 | 172.16.10.253 |
| VLAN 200 | 172.16.20.253 |

Switch leaf2:

cumulus@leaf2:mgmt:~$ net add vlan 100 ip address 172.16.10.253/24

cumulus@leaf2:mgmt:~$ net add vlan 200 vrf VRFA

cumulus@leaf2:mgmt:~$ net add vlan 200 ip address 172.16.20.253/24

cumulus@leaf2:mgmt:~$ net add vlan 200 vrf VRFA

cumulus@leaf1:mgmt:~$ net add vlan 10 ip address 172.16.10.252/24

cumulus@leaf1:mgmt:~$ net add vlan 10 vrf VRFA

cumulus@leaf1:mgmt:~$ net add vlan 20 ip address 172.16.20.252/24

cumulus@leaf1:mgmt:~$ net add vlan 20 vrf VRFA

- Access the leaf switches and configure the anycast default gateway’s VMAC and VIP. 
Use the following tables:

| Default gateway’s VIP | Default gateway’s VMAC |
| --- | --- |
| 172.16.10.254 | 00:00:5e:00:01:01 |
| 172.16.20.254 | 00:00:5e:00:01:02 |

Switch leaf1:

| Default gateway’s VIP | Default gateway’s VMAC |
| --- | --- |
| 172.16.10.254 | 00:00:5e:00:01:01 |
| 172.16.20.254 | 00:00:5e:00:01:02 |

Switch leaf2:

cumulus@leaf2:mgmt:~$ net add vlan 100 ip address-virtual 00:00:5e:00:01:01 172.16.10.254/24

cumulus@leaf2:mgmt:~$ net add vlan 200 ip address-virtual 00:00:5e:00:01:02 172.16.20.254/24

cumulus@leaf2:mgmt:~$ net commit

cumulus@leaf1:mgmt:~$ net add vlan 10 ip address-virtual 00:00:5e:00:01:01 172.16.10.254/24

cumulus@leaf1:mgmt:~$ net add vlan 20 ip address-virtual 00:00:5e:00:01:02 172.16.20.254/24

cumulus@leaf1:mgmt:~$ net commit

Please note:

The VMAC and VIP must be identical on both VTEPs for each VLAN in order to implement anycast default gateway and to allow proper inter-VXLAN routing.

Task 3: Verify L3-VNI configuration

cumulus@leaf1:mgmt:~$ net show evpn vni 4001

VNI: 4001

Type: L3

Tenant VRF: VRFA

Local Vtep Ip: 172.16.100.1

Vxlan-Intf: VNI-4001

SVI-If: vlan4001

State: Up

VNI Filter: none

System MAC: 44:38:39:00:00:12

Router MAC: 44:38:39:00:00:12

L2 VNIs: 10 20

Verify that the L3-VNI is up:

cumulus@leaf1:mgmt:~$ net show vrf vni

VRF                  VNI      VxLAN IF     L3-SVI        State Rmac

VRFA                 4001     VNI-4001     vlan4001      Up    44:38:39:00:00:12

Task 4: Test inter-VXLAN communication

- Use ‘ping’ and ‘traceroute’ utilities to verify end-to-end communication between hosts in different VNIs. 
For example, ping from host ‘host1’ in VNI-10 to ‘host2’ in VNI-20. 
In this case the egress switch leaf1 will perform routing between the source VNI and the L3-VNI, while the egress switch leaf2 will perform routing between L3-VNI and the destination VNI.

cumulus@host1:~$ ping 172.16.20.29

PING 172.16.20.29 (172.16.20.29) 56(84) bytes of data.

64 bytes from 172.16.20.29: icmp_seq=7 ttl=62 time=2.91 ms

64 bytes from 172.16.20.29: icmp_seq=8 ttl=62 time=3.44 ms

cumulus@host1:~$ traceroute 172.16.20.29

traceroute to 172.16.20.29 (172.16.20.29), 30 hops max, 60 byte packets

1  172.16.10.252 (172.16.10.252)  0.588 ms  0.546 ms  0.546 ms

2  172.16.20.253 (172.16.20.253)  2.656 ms  2.653 ms  2.652 ms

3  172.16.20.29 (172.16.20.29)  3.431 ms  3.415 ms  3.416 ms

## PRACTICE 9: EDITING ANSIBLE INVENTORY FILE

Practice objectives:

In this practice session you perform the initial configurations required for Ansible to start working with the group servers and switches.

- You will configure hosts and groups in your Ansible hosts file.

- You will use Ansible ping module to validate the configuration.

- Last, you will use Ansible Variables to refine the hosts configuration.

Task 1: Editing the Ansible Inventory (hosts) file

- Connect to the ‘oob-mgmt-server’ and Use VIM, or another text editor to edit the /etc/ansible/hosts file:

# vi /etc/ansible/hosts

# Default ansible hosts file

# comments begin with the '#' character

#   - Blank lines are ignored

#   - Groups of hosts are delimited by [header] elements

#   - hostnames or ip addresses are accepted

#   - A hostname/ip can be a member of multiple groups

~

~

~

⚠ to exit VIM:

- Press ESC

- Type ‘:’

- Type “q!” to exit without saving or “wq” to save and exit

~

~

:wq

To edit the file using VIM go to insert mode by typing ‘a’ 
(make sure the word “—INSERT --” appears at the end of the page).

~

~

~

~

~

-- INSERT --

Task 2: Adding servers to the Inventory (hosts) file

# Default ansible hosts file

# comments begin with the '#' character

#   - Blank lines are ignored

#   - Groups of hosts are delimited by [header] elements

#   - hostnames or ip addresses are accepted

#   - A hostname/ip can be a member of multiple groups

host1

host2

host3

host4

~

~

~

While in “INSERT” mode, add the servers host name to the hosts file.

Please note:

- Every line that starts with ‘#’ is considered a comment and can be deleted.

- Instead of configuring each server in a different line, you can use a REGEX expression to capture all group servers in one line # host[1:4]

Task 3: Testing Ansible connectivity using the “ping” module

# Default ansible hosts file

# comments begin with the '#' character

#   - Blank lines are ignored

#   - Groups of hosts are delimited by [header] elements

#   - hostnames or ip addresses are accepted

#   - A hostname/ip can be a member of multiple groups

host1

host2

host3

host4

~

~

~

:wq

Save and Exit the hosts file (type ESC, ‘:’, ‘wq’ and <enter>)

cumulus@oob-mgmt-server:~$ ansible host4 -m ping

host4 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

cumulus@oob-mgmt-server:~$

cumulus@oob-mgmt-server:~$ ansible host3 -m ping

host3 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

cumulus@oob-mgmt-server:~$

Validate the configuration by using the ping module

Task 4: Add servers to a host group

- Use VIM to edit the /etc/ansible/hosts file, and enter INSERT mode by typing ‘a’

# vi /etc/ansible/hosts

# Default ansible hosts file

# comments begin with the '#' character

#   - Blank lines are ignored

#   - Groups of hosts are delimited by [header] elements

#   - hostnames or ip addresses are accepted

#   - A hostname/ip can be a member of multiple groups

[servers]

host1

host2

host3

host4

~

~

~

Add all servers to a group called “servers”

- Exit VIM and use the Ansible “ping” module to test the new group configuration.

cumulus@oob-mgmt-server:~$ ansible switches -m ping

leaf2 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

leaf1 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

spine3 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

spine4 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

Task 5: Add Cumulus Linux switches to the inventory

- Use VIM to edit the /etc/ansible/hosts file, and enter INSERT mode by typing ‘a’

# vi /etc/ansible/hosts

# Default ansible hosts file

# comments begin with the '#' character

#   - Blank lines are ignored

#   - Groups of hosts are delimited by [header] elements

#   - hostnames or ip addresses are accepted

#   - A hostname/ip can be a member of multiple groups

[servers]

host[1:4] ansible_user=cumulus ansible_ssh_pass=Academy123

leaf1 ansible_user=cumulus ansible_ssh_pass=Academy123

leaf2 ansible_user=cumulus ansible_ssh_pass=Academy123

~

~

~

Add the leaf switches (‘leaf1’ and ‘leaf2’) to the inventory file, also add the necessary credentials (user and password)
# leaf1 ansible_user=cumulus ansible_ssh_pass=Academy123

cumulus@oob-mgmt-server:~$ ansible leaf1 -m ping

leaf1 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

Exit VIM and use the Ansible “ping” module to test Ansible connectivity to the switches.

Please note:

- You might encounter a warning regarding the Python interpreter on the switches, it can be ignored.

- If the ansible user or password are incorrect, you will get the following error:

leaf2 | UNREACHABLE! => {

"changed": false,

"msg": "Failed to connect to the host via ssh: Permission denied (publickey,password).",

"unreachable": true}

Task 6: Add Cumulus Linux switches to a host group

- Use VIM to edit the /etc/ansible/hosts file, and enter INSERT mode by typing ‘a’

# vi /etc/ansible/hosts

.

.

.

[servers]

host[1:4] ansible_user=cumulus ansible_ssh_pass=Academy123

[leaves]

leaf1 ansible_user=cumulus ansible_ssh_pass=Academy123

leaf2 ansible_user=cumulus ansible_ssh_pass=Academy123

~

~

~

Add the leaf switches to a group called “leaves”

.

.

.

[servers]

host[1:4] ansible_user=cumulus ansible_ssh_pass=Academy123

[leaves]

leaf1 ansible_user=cumulus ansible_ssh_pass=Academy123

leaf2 ansible_user=cumulus ansible_ssh_pass=Academy123

[spines]

spine3 ansible_user=cumulus ansible_ssh_pass=Academy123

spine4 ansible_user=cumulus ansible_ssh_pass=Academy123

~

~

~

Add the spine switches, same way the leaves were added.

.

.

.

[servers]

host[1:4] ansible_user=cumulus ansible_ssh_pass=Academy123

[leaves]

leaf1 ansible_user=cumulus ansible_ssh_pass=Academy123

leaf2 ansible_user=cumulus ansible_ssh_pass=Academy123

[spines]

spine3 ansible_user=cumulus ansible_ssh_pass=Academy123

spine4 ansible_user=cumulus ansible_ssh_pass=Academy123

[switches:children]

spines 
leaves

~

~

~

Add the leaves and spines to a group called “switches”.

cumulus@oob-mgmt-server:~$ ansible switches -m ping

leaf2 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

leaf1 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

spine3 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

spine4 | SUCCESS => {

"ansible_facts": {

"discovered_interpreter_python": "/usr/bin/python"

},

"changed": false,

"ping": "pong"

}

Exit VIM and use the Ansible “ping” module to test Ansible connectivity to the switches

Task 7: Add variables to be shared by the groups

.

.

.

[group_b_servers]

host[1:4] ansible_user=cumulus ansible_ssh_pass=Academy123

[leaves]

leaf1 ansible_user=cumulus ansible_ssh_pass=Academy123

leaf2 ansible_user=cumulus ansible_ssh_pass=Academy123

[spines]

spine3 ansible_user=cumulus ansible_ssh_pass=Academy123

spine4 ansible_user=cumulus ansible_ssh_pass=Academy123

[switches:children]

spines 
leaves

[switches:vars]

ansible_user=cumulus

ansible_ssh_pass=Academy123

~

~

~

Add the username and password as variables, to be shared among all ‘switches’ group members, then delete the definitions on each switch.

## PRACTICE 10: ANSIBLE PLAYBOOK

Practice objectives:

In this practice session you configure an Ansible playbook based on a familiar practice exercise

- You will configure the servers IP settings – an IP address, a subnet mask and a default gateway.

- You will configure a bridge on each of the Cumulus Linux switches in your group and add switch ports to the bridge.

- You will execute the playbook you wrote

- Last, you will use ‘ping’ utility to verify communication between servers in your group.

Topology used in this practice:

host4

host3

host2

host1

172.16.1.X

172.16.1.X

172.16.1.X

swp2
trunk

swp9
access

swp8
access

swp9
access

swp8
access

spine3

spine4

Eth2

swp2
trunk

swp1
trunk

swp1
trunk

leaf2

leaf1

172.16.1.X

Eth2

Eth2

Eth2

Task 1: Create a new Ansible playbook

- Access the Ansible controller server that was assigned to you, and create a new yaml file under the /etc/ansible/ directory

# touch /etc/ansible/labPlaybook.yaml

- Use VIM or another text editor to edit the /etc/ansible/labPlaybook.yaml file:

# vi /etc/ansible/labPlaybook.yaml

(the file should be empty)

- You should apply similar commands to devices in the group that was assigned to you.

- Every line that starts with ‘#’ is considered a comment and can be deleted.

Please note:

The tasks and commands that you will configure in the playbook are based on Practice3 in this lab guide.

Task 2: Resetting old configurations

- Add a new task to the playbook, the task purpose is to clear the old configurations before we apply our own. This task should be applied on the servers group.

- Add two additional sub-tasks under the task you configured:

  - Clear the current eth2 interfaces IP addresses in each server

  - Clear static routes in each server

# /etc/ansible/labPlaybook.yaml

- name: Clear server’s old configurations

hosts: servers

tasks:

- name: Clear eth2 interface existing IP

command: ifconfig eth2 0.0.0.0

- name: Clear static routes for network 172.30.0.0/16

command: ip route del 172.30.0.0/16

ignore_errors: yes

Please note:

Clearing an empty routing table might cause errors, and those errors are not relevant to the playbook execution, therefore should be ignored.
Please make sure that the “ignore_errors” value is set to “yes”.

- Add a new task to the playbook, the task purpose is to clear the old switch configurations before we apply our own. This task should be applied on the ‘switches’ group.

- Add one sub-task under the task you configured:

  - Use the NCLU module to clear all switch configurations with the net del all command.

# /etc/ansible/labPlaybook.yaml

.

.

.

- hosts: switches

tasks:

- name: Clear old switch configurations

nclu:

commands:

- del all

commit: true

Task 3: Configure switches according to the practice exercise

- Add a new task to the playbook, the task purpose is to apply configuration that 
all the switches share. This task should be applied on the switch group.

- add two sub-task under the task you configured, use NCLU module to apply:

  - configure trunks between the switches

# /etc/ansible/labPlaybook.yaml

.

.

.

- hosts: switches

tasks:

- name: Create a bridge and set inter switch links as trunk ports

nclu:

commands:

- add bridge bridge ports swp1-2

commit: false

- name: Create VLANs

nclu:

commands:

- add bridge bridge vids 2,3

commit: true

create the VLANs 2,3.

- Add a new task to the playbook, the task purpose is to apply configuration that all leaf switches share. This task should be applied on the leaves group.

- add one sub-task under the task you configured, use NCLU module to apply:

# /etc/ansible/labPlaybook.yaml

.

.

.

- hosts: leaves

tasks:

- name: Set the host-facing ports as access ports in VLAN 1 and COMMIT changes

nclu:

commands:

- add bridge bridge ports swp8-9

- add interface swp8 bridge access 2

- add interface swp9 bridge access 3

commit: true

set the host-facing ports as access ports, and associate each interface to it’s designated VLAN.

Task 4: Configure servers according to the practice exercise

# /etc/ansible/labPlaybook.yaml

.

.

.

- name: Configure host1

hosts: host1

tasks:

- name: Configure IP address

command: ifconfig eth2 172.30.12.18/24

- name: Configure host2

hosts: host2

tasks:

- name: Configure IP address

command: ifconfig eth2 172.30.13.19/24

- name: Configure host3

hosts: host3

tasks:

- name: Configure IP address

command: ifconfig eth2 172.30.12.28/24

- name: Configure host4

hosts: host4

tasks:

- name: Configure IP address

command: ifconfig eth2 172.30.13.29/24

Add 4 new tasks to the playbook, the tasks purpose is to configure the IP address on each server. Each task should be applied on each server.

# /etc/ansible/labPlaybook.yaml

.

.

.

- name: Add static route entry for VLAN 2

hosts: host1, host3

tasks:

- name: Add default gateway for 172.30.0.0/16

command: ip route add 172.30.0.0/16 dev eth2 via 172.30.12.254

- name: Add static route entry for VLAN 3

hosts: host2, host4

tasks:

- name: Add default gateway for 172.30.0.0/16

command: ip route add 172.30.0.0/16 dev eth2 via 172.30.13.254

Add 2 new tasks to the playbook, the tasks purpose is to configure static routes for each VLAN subnet. Each task should be applied on the servers associated with the VLAN.

Task 4: Executing the playbook

- Access the Ansible controller server that was assigned to you, and make sure that the host file is configured as follows

.

.

.

[servers]

host[1:4]

[leaves]

leaf1

leaf2

[spines]

spine3

spine4

[switches:children]

spines 
leaves

[switches:vars]

ansible_user=cumulus

ansible_ssh_pass=Academy123

~

~

~

# cat /etc/ansible/hosts

Please note:

- Configurations are demonstrated on group B servers and switches.
You should apply similar commands to devices in the group that was assigned to you.

- Every line that starts with ‘#’ is considered a comment and can be deleted.

- Execute the playbook you wrote

# sudo ansible-playbook -b /etc/ansible/labPlaybook.yaml

- Verify configuration on the servers and switches