CUMULUS NVUE: LAB GUIDE

Scope

This workbook covers configurations of network protocols and function using Nvidia Cumulus Linux as the Network Operating System. This workbook is based on the next generation CLI (NVUE). It is prepared for being used alongside the standard 3 session Cumulus Linux Boot Camp.

Audience

This workbook is intended for Technical Training students.

Objectives

By the end of this workbook, students will be able to:

- Configure basic switch functions with Cumulus Linux

- Configure layer 2 and layer 3 protocols with Cumulus Linux

- Verify configuration and connectivity.

- Monitor and troubleshoot networking related connectivity issues

Overview

Each student will be using the Nvidia Cumulus Air © platform, exercises in this workbook on a group of devices (four servers and four switches).

Notice

Please follow the instructions below carefully to successfully complete the practice. 
If you encounter technical issues, please contact the Nvidia Networking Academy team:

academy-support@nvidia.com

Release Date and Disclaimer

Revision 1.0 – April 2022 (Based on CL 5.0.1 Software release)

The lab was created by using Cumulus VX, the behavior/functionality of a physical environment might differ.

Good Luck, 
NVIDIA Academy team

TABLE OF CONTENTS

Prerequisites and Guidelines	3

Academy Lab Topology	4

Academy Lab Access	6

PRACTICE UNITS

Practice-01: Preparing the lab and first steps	8

Practice-02: Basic Switch Functions	22

Practice-03: VLANs and Trunking	29

Practice-04: Configuring MLAG and VRR	35

Practice-05: Configuring BGP Unnumbered	49

Practice-06: Configuring VXLAN with EVPN	59

Practice-07: Distributed Symmetric VXLAN Routing	65

# PREREQUISITES AND GUIDELINES

Please perform and review the following steps before you start:

- Enter the Cumulus Air web page : https://air.nvidia.com/Login
Click “GET STARTED” button.

- If you have already created an account, 
use your credentials to Login.

- To sign up for the first time, click “Register” 
and fill in your details.

Once completed, a confirmation email will 
be sent, open it to activate your new account.

- Once you are logged in, you will reach the “Cumulus in The Cloud” dashboard. 
Wait for the lab to be Loaded.

- Click on the “Academy ILT CL5.0.1” label/name to access your lab.

## ACADEMY LAB TOPOLOGY

Every student is using her/his own individual lab.

The training lab is organized in the following topology:

The lab layout resembles 100% the former NCLU based Cumulus Linux boot camp lab layout. For most exercises the links between the two leaf’s are NOT used.

Four switches (leaf1, leaf2, spine3, and spine4) are running CL 5.0.1.

cumulus@leaf1:mgmt:~$ net show ver

NCLU_VERSION=1.0-cl5.0.0u11

DISTRIB_ID="Cumulus Linux"

DISTRIB_RELEASE=5.0.1

DISTRIB_DESCRIPTION="Cumulus Linux 5.0.1"

The four servers (host1, host2, host3, and host4) are running Ubuntu 20.04.

cumulus@host1:~$ lsb_release -a

No LSB modules are available.

Distributor ID: Ubuntu

Description:    Ubuntu 20.04.4 LTS

Release:        20.04

Codename:       focal

The out-of-band (OOB) infrastructure is provided via:

An oob-mgmt-switch, which is transparent during the entire labs, and is running CL 4.4.0.

The Jump-host (oob-mgmt-server) is running Ubuntu 18.04 and provides the services like DHCP and Orchestration (Ansible), next to others:

cumulus@oob-mgmt-server:/home/ubuntu$ lsb_release -a

No LSB modules are available.

Distributor ID: Ubuntu

Description:    Ubuntu 18.04.6 LTS

Release:        18.04

Codename:       bionic

SSH access:

The user-account for the oob-mgmt-server when accessing via SSH might be “nvidia”. We will use the “cumulus” user, please change from user “nvidia” to user “cumulus” if needed.

## ACADEMY LAB ACCESS

Views

The simulation environment offers two views: standard and advanced.
Via a button on the upper right corner:

you can switch between the views. We recommend using the “Advanced View”.

Console-GUI
The console-GUI offers a connection to the oob-mgmt-server. 
Using “cumulus” as the user and “Academy123” as the passwords allows access. 
From here you can ssh directly to the nodes e.g., ssh leaf1

In the upper right corner you find the icon to pop-out the Console-GUI, which allows you to open multiple connections via the “plus icon” e.g. to the oob-mgmt-server and leaf1:

- When the login prompt appears, enter the username – “cumulus”

- When the password prompt appears, enter the password – “Academy123” and press Enter.

- You should now be prompted with the node’s name. This indicates that you have successfully accessed the node.

## PRACTICE 1: PREPARING THE LAB AND FIRST STEPS

netd and nvued - Daemons:

In this Lab we use CL 5.0 or newer for our “production” network (leaf1, leaf2, spine3, and spine4). Releases 4.3 and 4.4 used in former labs, could have enabled NVUE but had to have NCLU disabled. With CL 5.0 both daemons for NVUE and NCLU are active, the later one for providing a complete set of show commands, all starting with 
$ net show …

General:
All tasks are executed with the user cumulus unless explicitly stated.

Practice Objectives:

In this practice session you will become familiar with the Cumulus Linux NVUE cli:

- You will use ‘nv set’ and ‘nv unset’ commands to change the configuration.

- You will use ‘nv config apply’ command to apply configuration changes.

- To save the configuration we will use use ‘nv config save’.

- You will use ‘nv config diff’ commands to compare revisions.

- Lastly, you will use ‘nv config patch’ and use ‘nv config replace’ to modify the pending configurations.

- To view items, we use use ‘nv show’.

Task-01

cumulus@leaf1:mgmt:~$ sudo systemctl status netd

netd.service - Network Command Line Utility Daemon

Loaded: loaded (/lib/systemd/system/netd.service; enabled; vendor preset: enabled)

Active: active (running) since Mon 2022-03-07 08:48:57 UTC; 2 weeks 0 days ago

Main PID: 975 (python)

Memory: 52.9M

CGroup: /system.slice/netd.service

└─975 /usr/bin/python -O /usr/sbin/netd -d

Please ssh to at least one node and verify that both daemons are running.

cumulus@leaf1:mgmt:~$ sudo systemctl status nvued

nvued.service - NVIDIA User Experience Daemon

Loaded: loaded (/lib/systemd/system/nvued.service; enabled; vendor preset: enabled)

Active: active (running) since Mon 2022-03-07 08:48:50 UTC; 2 weeks 0 days ago

Main PID: 411 (python3)

Memory: 222.8M

CGroup: /system.slice/nvued.service

└─411 /usr/bin/python3 /usr/sbin/nvued –daemon

Configuration tasks are not feasible via NCLU. 
In other words, we use NCLU only for providing show commands.

cumulus@leaf1:mgmt:~$ net show interface

State  Name  Spd  MTU    Mode      LLDP                    Summary

-----  ----  ---  -----  --------  ----------------------  --------------------------

UP     lo    N/A  65536  Loopback                          IP: 127.0.0.1/8

lo                                                  IP: ::1/128

UP     eth0  1G   1500   Mgmt      oob-mgmt-switch (swp2)  Master: mgmt(UP)

eth0                                                IP: 192.168.200.2/24(DHCP)

UP     swp1  1G   9216   Default   spine3 (swp1)

UP     swp2  1G   9216   Default   spine4 (swp2)

UP     swp5  1G   9216   Default   leaf2 (swp5)

UP     swp6  1G   9216   Default   leaf2 (swp6)

UP     swp8  1G   9216   Default

UP     swp9  1G   9216   Default

UP     mgmt  N/A  65536  VRF                               IP: 127.0.0.1/8

mgmt                                                IP: ::1/128

Task-02

This output suggests that a non-default configuration is in place. 
Interfaces swp are not configured or available with factory settings.

We will “clean-up” any legacy configurations before we start.

Task-03

SSH to all cumulus nodes and verify if the file /etc/nvue.d/startup.yaml is present.

By default, this file (as of CL 5.0) is not present. If present remove the file.

Apply the empty config to all nodes:

$ nv config apply empty

This also removes the hostname. Thus, apply the individual hostname via 
$ nv set system hostname <name>

The syntax has changed between CL 4.4 and CL 5.0, so if you happen to have an older NVUE config file, please verify, and change the hostname configuration.

Apply your cleaned configuration via
$ nv config apply

Task-04

Host setup

The lab is using four hosts (host1, host2, host3, and host4) running Linux (Ubuntu 20.04). The Interface Manager by default is Netplan.

In case you are new to Linux and/or Netplan, please consider consulting external resources like:

https://netplan.io/examples/

or ask your instructor for a quick-live-demo/introduction.

Alternatively, to Netplan you can configure the hosts via:

- ifconfig

- ip(route2)

Both options create the same functionality for our use-cases but both are not persistent and might result in more time being consumed during the class.

However, ip(route2) provides the most configuration options compared to both ifconfig and netplan.

- Our order of preference:

- Netplan

- Ip(route2)

- ifconfig

Each hosts use two interfaces:

- Eth0	for OOB Management, this interface should NOT be changed

- Eth2	this interface connecting to the leaf switches and will be configured during class

Task-06

Enable all switch interfaces

Verify which interfaces are available on all four cumulus nodes:

cumulus@leaf1:mgmt:~$ net show int all

State  Name  Spd  MTU    Mode           LLDP                    Summary

-----  ----  ---  -----  -------------  ----------------------  --------------------------

UP     lo    N/A  65536  Loopback                               IP: 127.0.0.1/8

lo                                                       IP: ::1/128

UP     eth0  1G   1500   Mgmt           oob-mgmt-switch (swp2)  Master: mgmt(UP)

eth0                                                     IP: 192.168.200.2/24(DHCP)

ADMDN  swp1  N/A  9216   NotConfigured

ADMDN  swp2  N/A  9216   NotConfigured

ADMDN  swp5  N/A  9216   NotConfigured

ADMDN  swp6  N/A  9216   NotConfigured

ADMDN  swp8  N/A  9216   NotConfigured

ADMDN  swp9  N/A  9216   NotConfigured

UP     mgmt  N/A  65536  VRF                                    IP: 127.0.0.1/8

mgmt                                                     IP: ::1/128

We can see for example on leaf1 that the interfaces swp1, swp2, swp5, swp6, swp8, and swp9 are “available” but in ADMINISTRATIVE DOWN state.

Enable all ADMDN interfaces (ADMDN == ADMIN DOWN):

cumulus@leaf1:mgmt:~$ nv set interface swp1,swp2,swp5,swp6,swp8,swp9 cumulus@leaf1:mgmt:~$ nv config diff - set: interface: swp1-2,5-6,8-9: type: swp

Is your configuration active? … “activate” your configuration and verify.

Hosts

Configure your hosts with the following IPv4 addresses, we don’t need a default gateway right now:

Host1(eth2): 172.16.1.1/24 Host2(eth2): 172.16.1.2/24 Host3(eth2): 172.16.1.3/24 Host4(eth2): 172.16.1.4/24

Task-07

Verify your link-layer neighbors (LLDP)

Verify that your switches are connected correctly according to the lab layout by using LLDP:

cumulus@leaf1:mgmt:~$ net show lldp LocalPort Speed Mode RemoteHost RemotePort --------- ----- ------- --------------- ----------------- eth0 1G Mgmt oob-mgmt-switch swp2 swp1 1G Default spine3 swp1 swp2 1G Default spine4 swp2 swp5 1G Default leaf2 swp5 swp6 1G Default leaf2 swp6 swp8 1G Default host1 44:38:39:00:00:11 swp9 1G Default host2 44:38:39:00:00:13

Task-08

Optional

It is beneficial to create maybe old-fashioned on a piece of paper a network documentation with the information about Layer 1 and Layer 2 to be used as a reference during class.

Task-09

Optional PTM

PTM is the abbreviation for Prescriptive Topology Manager, a service to verify if the intended layout correlates with the actual layout (both with a simulation or physical network).

Verify that your PTM daemon is running:

cumulus@leaf1:mgmt:/etc/ptm.d$ sudo systemctl status ptmd ptmd.service - Prescriptive Topology Manager (PTM) Daemon. Loaded: loaded (/lib/systemd/system/ptmd.service; enabled; vendor preset: enabled) Drop-In: /etc/systemd/system/ptmd.service.d └─ptmd.conf Active: active (running) since Mon 2022-03-07 08:48:58 UTC; 2 weeks 1 days ago Docs: man:ptmd(8) man:ptmctl(8) Main PID: 1093 (ptmd) Memory: 932.0K CGroup: /system.slice/ptmd.service └─1093 /usr/sbin/ptmd -l INFO

PTM is using a file called topology.dot which needs to be stored in the 
ptm.d directory:

cumulus@leaf1:mgmt:/etc/ptm.d$ ls bfd-sess-down bfd-sess-up if-topo-fail if-topo-pass

here in this example this file is missing.

Create a topology.dot file on one or multiple cumulus nodes. 
You can use the following 3-line template as a starting point:

graph "<your name>" { "spine3":"swp1" -- "leaf1":"swp1" }

Once in place restart ptm and verify. For leaf1 the expected result is:

cumulus@leaf1:mgmt:~$ ptmctl ----------------------------------------- port cbl BFD BFD BFD BFD status status peer local type ----------------------------------------- swp5 pass N/A N/A N/A N/A swp6 pass N/A N/A N/A N/A swp8 pass N/A N/A N/A N/A swp9 pass N/A N/A N/A N/A swp1 pass N/A N/A N/A N/A swp2 pass N/A N/A N/A N/A cumulus@leaf1:mgmt:~$ net show lldp LocalPort Speed Mode RemoteHost RemotePort --------- ----- ------- --------------- ----------------- eth0 1G Mgmt oob-mgmt-switch swp2 swp1 1G Default spine3 swp1 swp2 1G Default spine4 swp2 swp5 1G Default leaf2 swp5 swp6 1G Default leaf2 swp6 swp8 1G Default host1 44:38:39:00:00:11 swp9 1G Default host2 44:38:39:00:00:13

Task-10

Environment

Verify the FAN, LED, and SENSOR information, on a virtual switch dummy information is made available for the show commands:

cumulus@leaf1:mgmt:~$ net show system sensors Fan1 (Fan Tray 1, Fan 1 ): OK Fan2 (Fan Tray 1, Fan 2 ): OK Fan3 (Fan Tray 2, Fan 1 ): OK Fan4 (Fan Tray 2, Fan 2 ): OK Fan5 (Fan Tray 3, Fan 1 ): OK Fan6 (Fan Tray 3, Fan 2 ): OK PSU1 : OK PSU2 : OK PSU1Fan1 (PSU1 Fan ): OK PSU1Temp1 (PSU1 Temp Sensor ): OK PSU2Fan1 (PSU2 Fan ): OK PSU2Temp1 (PSU2 Temp Sensor ): OK Temp1 (Board Sensor near CPU ): OK Temp2 (Board Sensor Near Virtual Switch ): OK Temp3 (Board Sensor at Front Left Corner ): OK Temp4 (Board Sensor at Front Right Corner ): OK Temp5 (Board Sensor near Fan ): OK

cumulus@leaf1:mgmt:~$ net show system leds Power: green Fan: green System: green

cumulus@leaf1:mgmt:~$ nv show platform environment operational applied description -------- ---------------------------------- ------- -------------------------- [fan] Fan1 The fans on the switch. [fan] Fan2 [fan] Fan3 [fan] Fan4 [fan] Fan5 [fan] Fan6 [fan] PSU1Fan1 [fan] PSU2Fan1 [led] Fan The LEDs on the switch. [led] Power [led] System [psu] PSU1 The PSUs on the switch. [psu] PSU2 [sensor] Board Sensor Near Virtual Switch The sensors on the switch. [sensor] Board Sensor at Front Left Corner [sensor] Board Sensor at Front Right Corner [sensor] Board Sensor near CPU [sensor] Board Sensor near Fan [sensor] PSU1 Temp Sensor [sensor] PSU2 Temp Sensor

cumulus@leaf1:mgmt:~$ nv show platform hardware operational applied description ------------- ----------------- ------- ----------------------------------------- base-mac 44:38:39:00:00:1C The base mac address provided by eeprom manufacturer Cumulus The platform's manufacturer memory 1.88 GB Hardware RAM model VX The platform's model identifier part-number 5.0.1 System part number product-name VX Product Name serial-number 44:38:39:00:00:1c System serial number system-mac 44:38:39:00:00:23 The MAC provided by eeprom for system-mac cumulus@spine3:mgmt:~$ net show ver NCLU_VERSION=1.0-cl5.0.0u11 DISTRIB_ID="Cumulus Linux" DISTRIB_RELEASE=5.0.1 DISTRIB_DESCRIPTION="Cumulus Linux 5.0.1"

Task-11

Remove interfaces

Remove the direct interfaces between the leaf switches.

$ nv unset interface swp5-6 $ nv config apply

Task-12

Verify the configuration history

$ nv config history

Example output:

cumulus@leaf1:mgmt:~$ nv config history - apply-id: n/9 apply-meta: method: CLI reason: Config update rev_id: changeset/cumulus/2022-03-22_12.12.01_AWD3 state_controls: {} user: cumulus date: '2022-03-22T12:12:16+00:00' message: Config update by cumulus via CLI ref: apply/2022-03-22_12.12.13_AWD4/done - apply-id: n/8 apply-meta: method: CLI reason: Config update rev_id: startup state_controls: {} user: root date: '2022-03-22T10:52:36+00:00' message: Config update by root via CLI ref: apply/2022-03-22_10.52.34_AWD2/done

Identify the last two apply-id’s, here n/9 and n/8.
n/9 includes the unsetting of swp5-6 and n/8 should include them as the state before unsetting.

Verify that the interfaces swp5-6 are not set on your leaf switches:

cumulus@leaf1:mgmt:~$ net show int State Name Spd MTU Mode LLDP Summary ----- ---- --- ----- -------- ------------------------- -------------------------- UP lo N/A 65536 Loopback IP: 127.0.0.1/8 lo IP: ::1/128 UP eth0 1G 1500 Mgmt oob-mgmt-switch (swp2) Master: mgmt(UP) eth0 IP: 192.168.200.2/24(DHCP) UP swp1 1G 9216 Default spine3 (swp1) UP swp2 1G 9216 Default spine4 (swp2) UP swp8 1G 9216 Default host1 (44:38:39:00:00:11) UP swp9 1G 9216 Default host2 (44:38:39:00:00:13) UP mgmt N/A 65536 VRF IP: 127.0.0.1/8 mgmt IP: ::1/128

cumulus@leaf1:mgmt:~$ nv config apply n/8 applied cumulus@leaf1:mgmt:~$ net show int State Name Spd MTU Mode LLDP Summary ----- ---- --- ----- -------- ------------------------- -------------------------- UP lo N/A 65536 Loopback IP: 127.0.0.1/8 lo IP: ::1/128 UP eth0 1G 1500 Mgmt oob-mgmt-switch (swp2) Master: mgmt(UP) eth0 IP: 192.168.200.2/24(DHCP) UP swp1 1G 9216 Default spine3 (swp1) UP swp2 1G 9216 Default spine4 (swp2) UP swp5 1G 9216 Default leaf2 (swp5) UP swp6 1G 9216 Default leaf2 (swp6) UP swp8 1G 9216 Default host1 (44:38:39:00:00:11) UP swp9 1G 9216 Default host2 (44:38:39:00:00:13) UP mgmt N/A 65536 VRF IP: 127.0.0.1/8 mgmt IP: ::1/128

Task 13: Save the configuration

- Make the applied configuration persistent (on all four switches):
$ nv config save

cumulus@leaf1:mgmt:~$ nv config save

saved

Optional (more advanced) tasks:

Use a single line instruction (CLI) to make the config persistent on all four switches at once.

What would be your suggestions, please discuss in class during lab debriefing.

Task 14: Compare

- set:

system:

hostname: leaf1

interface:

swp1-2,5,8-9:

type: swp

Create on leaf1 a file in yaml syntax to add the interfaces swp5 and name it /home/cumulus/replace.yaml which includes the active configuration as a base:

$ nv config show > /home/cumulus/replace.yaml

Select an editor (e.g. nano or vi) and add Interface swp5

$ nano replace.yaml

- Replace the pending configuration

$ nv config replace ./replace.yaml

- Verify the pending and the applied revisions
$ nv config diff empty

$ nv config diff empty applied

cumulus@leaf1:mgmt:~$ nv config diff

- set:

interface:

swp5:

type: swp

cumulus@leaf1:mgmt:~$ nv config diff empty

- set:

system:

hostname: leaf1

interface:

swp1-2,5,8-9:

type: swp

Apply the configuration and verify that swp5 is available.

cumulus@leaf1:mgmt:~$ net show int swp5 Name MAC Speed MTU Mode -- ---- ----------------- ----- ---- ------- UP swp5 44:38:39:00:00:0d 1G 9216 Default

- Copy the replace.yaml file, rename it to patch.yaml add the missing interface swp6.

Within patch.yaml remove the hostname section.

Add this set of information to the pending revision.
Use cl config diff commands to compare and verify.
Apply and save the new configuration.

$ cp replace.yaml patch.yaml $ nano patch.yaml $ nv config patch patch.yaml $ nv config diff empty $ nv config diff startup $ nv config diff applied $ nv config apply $ nv config save

## PRACTICE 2: BASIC SWITCH FUNCTIONS

Practice Objectives:

In this practice session you will create and verify IP connectivity between servers in the lab.

- You will configure the servers IP settings – an IP address, a subnet mask and a default gateway.

- You will configure a bridge on each of the Cumulus Linux switches and add switch ports to the bridge.

- You will use ‘ping’ utility to verify communication between servers in your group.

- Last, you will observe how the switch forwarding database – the MAC address table – is built and maintained.

Topology Used in this Practice:

Task 1

Configure Servers IP Settings

- Access the servers and check interface ’eth2’ MAC and IP settings:

cumulus@host1:~$ ip -br a

lo               UNKNOWN        127.0.0.1/8 ::1/128

eth0             UP             192.168.200.6/24 fe80::4638:39ff:fe00:24/64

eth2             UP             172.16.1.1/24 fe80::4638:39ff:fe00:11/64

$ ip address show dev eth2

or list all interfaces:

- If they differ from the below table, correct them (/24 subnet):

| Practice Lab Servers Properties |  |
| --- | --- |
| Server | ‘eth2’ IP Address |
| host1 | 172.16.1.1 |
| host2 | 172.16.1.2 |
| host3 | 172.16.1.3 |
| host4 | 172.16.1.4 |

Task 2

Clean up and configure a Bridge

- Access the switches and reset configuration:

$ nv config apply empty -y $ nv set system hostname [leaf1¦leaf2¦spine3¦spine4] $ nv config apply -y $ nv config save

cumulus@leaf1:mgmt:~$ nv config save

saved

- On all four switches – leaves and spines – create or enable a bridge named br_default, add VLAN10.
Set the inter switch links (swp1 and swp2) as trunk ports:

$ nv set bridge domain br_default vlan 10 $ nv set interface swp1-2 bridge domain br_default

On the leaf switches only – leaf1 and leaf2 - set the host-facing ports, swp8 and swp9, as access ports in VLAN 10:

$ nv set interface swp8-9 bridge domain br_default access 10

Optional (more advanced) tasks:

Verify in the sys-file-system the resulting bridge type (traditional or vlan-aware-bridge (VAB).

Verify in the /e/n/i file the resulting bridge type.

Verify via ifquery the resulting bridge type.

cumulus@leaf1:mgmt:~$ nv config apply

applied

Apply the pending revision:
$ nv config apply

Verification:

All hosts are configured with the same subnet 172.16.1.0/24. You should be able to reach from every host every other host. No default gateway is needed.

- Verify configuration:
$ nv config diff startup applied
$ nv config show

cumulus@leaf1:mgmt:~$ nv config show

- set:

bridge:

domain:

br_default:

vlan:

'10': {}

system:

hostname: leaf1

interface:

swp1-2:

bridge:

domain:

br_default: {}

swp1-2,8-9:

type: swp

swp8-9:

bridge:

domain:

br_default:

access: 10

- Verify bridge mac-table:

$ nv show bridge domain br_default mac-table

$ net show bridge

cumulus@leaf1:mgmt:~$ nv show bridge domain br_default mac-table

age  bridge-domain  entry-type  interface   last-update  mac                src-vni  vlan  vni  Summary

----  ---  -------------  ----------  ----------  -----------  -----------------  -------  ----  ---  -------

+ 0   27   br_default                 swp1        90           44:38:39:00:00:05           1

+ 1   93   br_default     permanent   swp1        93           44:38:39:00:00:06

+ 10  93   br_default     permanent   br_default  93           44:38:39:00:00:23

+ 2   30   br_default                 swp2        88           44:38:39:00:00:17           10

+ 3   28   br_default                 swp2        88           44:38:39:00:00:15           10

+ 4   28   br_default                 swp2        89           44:38:39:00:00:0b           1

+ 5   93   br_default     permanent   swp2        93           44:38:39:00:00:0c

+ 6   1    br_default                 swp8        88           44:38:39:00:00:11           10

+ 7   93   br_default     permanent   swp8        93           44:38:39:00:00:12

+ 8   1    br_default                 swp9        88           44:38:39:00:00:13           10

+ 9   93   br_default     permanent   swp9        93           44:38:39:00:00:14

cumulus@leaf1:mgmt:~$ net show bridge macs

VLAN      Master      Interface   MAC                TunnelDest  State      Flags  LastSeen

--------  ----------  ----------  -----------------  ----------  ---------  -----  --------

1         br_default  swp1        44:38:39:00:00:05                                00:00:29

1         br_default  swp2        44:38:39:00:00:0b                                00:00:29

10        br_default  swp2        44:38:39:00:00:15                                00:00:53

10        br_default  swp2        44:38:39:00:00:17                                00:01:02

10        br_default  swp8        44:38:39:00:00:11                                00:00:02

10        br_default  swp9        44:38:39:00:00:13                                00:00:02

untagged  br_default  br_default  44:38:39:00:00:23              permanent         00:03:05

untagged  br_default  swp1        44:38:39:00:00:06              permanent         00:03:05

untagged  br_default  swp2        44:38:39:00:00:0c              permanent         00:03:05

untagged  br_default  swp8        44:38:39:00:00:12              permanent         00:03:05

untagged  br_default  swp9        44:38:39:00:00:14              permanent         00:03:05

- Verify the bridge via the native Linux commands:

$ bridge fdb

$ bridge link

cumulus@leaf1:mgmt:~$ bridge vlan

port    vlan ids

swp1     1 PVID Egress Untagged

10

swp2     1 PVID Egress Untagged

10

swp8     10 PVID Egress Untagged

swp9     10 PVID Egress Untagged

br_default      None

cumulus@leaf1:mgmt:~$ bridge link

3: swp1: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 master br_default state forwarding priority 8 cost 4

4: swp2: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 master br_default state forwarding priority 8 cost 4

7: swp8: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 master br_default state forwarding priority 8 cost 4

8: swp9: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 master br_default state forwarding priority 8 cost 4

cumulus@leaf1:mgmt:~$ bridge fdb

44:38:39:00:00:05 dev swp1 vlan 1 master br_default

44:38:39:00:00:06 dev swp1 master br_default permanent

44:38:39:00:00:17 dev swp2 vlan 10 master br_default

44:38:39:00:00:15 dev swp2 vlan 10 master br_default

44:38:39:00:00:0b dev swp2 vlan 1 master br_default

44:38:39:00:00:0c dev swp2 master br_default permanent

44:38:39:00:00:11 dev swp8 vlan 10 master br_default

44:38:39:00:00:12 dev swp8 master br_default permanent

44:38:39:00:00:13 dev swp9 vlan 10 master br_default

44:38:39:00:00:14 dev swp9 master br_default permanent

44:38:39:00:00:23 dev br_default master br_default permanent

$ bridge vlan

Task 3

Observe a Switch’s Forwarding Database

cumulus@host1:~$ ip -br l

lo               UNKNOWN        00:00:00:00:00:00 <LOOPBACK,UP,LOWER_UP>

eth0             UP             44:38:39:00:00:24 <BROADCAST,MULTICAST,UP,LOWER_UP>

eth2             UP             44:38:39:00:00:11 <BROADCAST,MULTICAST,UP,LOWER_UP>

Identify the MAC addresses of all four servers/hosts in your setup.

verify that the eth2 MAC addresses is stored within the FDB/mac-address-tables of your switches

cumulus@host1:~$ ping 172.16.1.3

PING 172.16.1.3 (172.16.1.3) 56(84) bytes of data.

64 bytes from 172.16.1.3: icmp_seq=1 ttl=64 time=2.99 ms

64 bytes from 172.16.1.3: icmp_seq=2 ttl=64 time=2.28 ms

Use ‘ping’ from one server to another server in the lab. 
For example, ping from server host1 to server host3.

## PRACTICE 3: VLANs AND TRUNKING

Practice Objectives:

In this practice session you will configure and verify VLANs and trunking:

- You will configure two new VLANs and assign switch ports connected to servers to the configured VLANs.

- You will configure SVIs to allow inter-VLAN communication.

Topology Used in this Lab:

Task 1: Configuring VLANs and Trunking

- Access the leaf and spine switches and reset configuration:

$ nv config apply empty

$ nv set system hostname [leaf1,leaf2,spine3,spine4]

$ nv config apply

- On all four switches – spines and leaves - create a vlan-aware-bridge, 
two vlans (2 and 3) and set the inter switch links, swp1 and swp2, as trunk ports:

$ nv set bridge domain br_default vlan 2-3

$ nv set interface swp1-2 bridge domain br_default

cumulus@leaf1:mgmt:~$ nv show bridge domain br_default vlan

multicast.snooping.querier.source-ip  ptp.enable  Summary

---  ------------------------------------  ----------  -------

+ 2  0.0.0.0                               off

+ 3  0.0.0.0                               off

Verify if vlan 1, without explicitly configuration, is used by default and for which use case:

cumulus@leaf1:mgmt:~$ net show bridge vlan

Interface  VLAN  Flags

---------  ----  ---------------------

swp1       1     PVID, Egress Untagged

2-3   []

swp2       1     PVID, Egress Untagged

2-3   []

Optionally, verify that LLDP is running between the switches. Following example shows spine3 view after configuring leaf1 but before configuring leaf2.

$ sudo lldpcli

[lldpcli] # show neighbors summary

-------------------------------------------------------------------------------

LLDP neighbors:

-------------------------------------------------------------------------------

Interface:    eth0, via: LLDP

Chassis:

ChassisID:    mac 44:38:39:00:00:19

SysName:      oob-mgmt-switch

Port:

PortID:       ifname swp4

PortDescr:    swp4

TTL:          120

-------------------------------------------------------------------------------

Interface:    swp1, via: LLDP

Chassis:

ChassisID:    mac 44:38:39:00:00:1c

SysName:      leaf1

Port:

PortID:       ifname swp1

PortDescr:    swp1

TTL:          120

-------------------------------------------------------------------------------

$ sudo lldpcli show neighbors

- On leaf switches only leaf1 and leaf2 set the host-facing ports, swp8 and swp9, as access ports and associate them to the appropriate VLAN: 
interface swp8 in VLAN2 and interface swp9 in VLAN3
$ nv set interface swp9 bridge domain br_default access 3

cumulus@leaf1:mgmt:~$ nv config apply

Commit changes and verify:

cumulus@leaf1:mgmt:~$ nv set interface swp8 bridge domain br_default access 2 cumulus@leaf1:mgmt:~$ nv set interface swp9 bridge domain br_default access 3

Verify VLANs configuration:

$ bridge vlan

cumulus@leaf1:mgmt:~$ bridge vlan

port    vlan ids

swp1     1 PVID Egress Untagged

2

3

swp2     1 PVID Egress Untagged

2

3

swp8     2 PVID Egress Untagged

swp9     3 PVID Egress Untagged

Please note:

- Access ports are shown with a single line representing the VLAN associated to the port.

- Trunk ports are shown with multiple lines representing the VLANs associated with the trunk port.

Task 2

Servers’ IP settings

Access the servers and configure an IP address for interface ‘eth2’ according to the table below.

Servers in the same VLAN will be configured with IP addresses in the same subnet, hence they will be able to communicate over the layer 2 network.

| VLAN ID | Server | ‘eth2’ IP Address |
| --- | --- | --- |
| VLAN 2 | host1 | 172.16.2.18/24 |
| VLAN 3 | host2 | 172.16.3.19/24 |
| VLAN 2 | host3 | 172.16.2.28/24 |
| VLAN 3 | host4 | 172.16.3.29/24 |

- Configure if needed the server’s IP address and subnet mask according to the table above.

- Verify a static route entry to network 172.16.0.0/16 via the default gateway’s address. Use the default gateway address in the following table:

| VLAN ID | Default gateway address |
| --- | --- |
| VLAN 2 | 172.16.2.254/24 |
| VLAN 3 | 172.16.3.254/24 |

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

a non-persistent setting can be achieved via ip(route2) or you might consider to configure via netplan.
$ sudo ip route add <NET_ADDRESS/MASK> dev <DEV> via <IP>

Question: how would you remove a static route you might have set incorrectly before?

[cumulus@host1 ~]# ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=4.11 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=2.58 ms

Use ‘ping’ to check communication between servers in the same VLAN. 
For example, servers host1 and host3.

Task 3: Configuring SVIs for inter-VLAN routing

- On switch spine3 configure two SVIs (Switch VLAN Interfaces) that will be used for routing between VLAN2 and VLAN3:

  - Interface vlan2 will serve as the default gateway for VLAN2

  - Interface vlan3 will serve as the default gateway for VLAN3

Use the following table for IP address assignment:

| Interface vlan 2 | 172.16.2.254/24 |
| --- | --- |
| Interface vlan 3 | 172.16.3.254/24 |

$ nv set interface vlan2 ip address 172.16.2.254/24

$ nv set interface vlan3 ip address 172.16.3.254/24

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip address 172.16.2.254/24

cumulus@spine3:mgmt:~$ nv set interface vlan3 ip address 172.16.3.254/24

cumulus@spine3:mgmt:~$ nv config apply

[cumulus@host1 ~]# ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=0.155 ms

64 bytes from 172.16.23.19: icmp_seq=2 ttl=63 time=0.192 ms64 bytes from 172.16.23.254: icmp_seq=2 ttl=64 time=0.362 ms

[cumulus@host1 ~]#  traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.340 ms  1.633 ms  1.607 ms

2  172.16.3.19 (172.16.3.19)  4.043 ms  4.023 ms  3.999 ms 2  172.16.3.19 (172.16.3.19)  0.155 ms  0.152 ms  0.137

Use ‘ping’ and ‘traceroute’ utilities to verify communication between hosts in different VLANs. For example, ping from server ‘host1’ in VLAN2 to server ‘host2’ in VLAN3.

## PRACTICE 4: CONFIGURING MLAG and VRR

Practice Objectives:

In this practice session you will configure the spine switches, spine3 and spine4, with MLAG and VRR towards the leaf switches.

- MLAG will make the spine switches to look and behave like a single Layer 2 switch towards the Layer 2 network.

- VRR will make the spine switches to look and behave like a single router providing default gateway redundancy.

- Interface ‘clag 1’ will aggregate swp1 on spine3 and swp2 on spine4 connected to switch leaf1. 
Switch leaf1 will be configured with a regular LAG.

- Interface ‘clag 2’ will aggregate swp2 on spine3 and swp1 on spine4 connected to switch leaf2. 
Switch leaf2 will be configured with a regular LAG.

Topology used in this lab:

Task 1: LAG Configuration – Leaf Switches

- Access the leaf switches leaf1 and leaf2 and reset configuration:

$ nv config apply empty

$ nv set system hostname [leaf1|leaf2]

- Create a bridge
$ nv set bridge domain br_default

- Create a bond named ‘BOND_TO_SPINES’, where bonds slaves are interfaces swp1 and swp2.

$ nv set interface BOND_TO_SPINES type bond

$ nv set interface BOND_TO_SPINES bond member swp1-2

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

cumulus@leaf1:mgmt:~$ net show interface bonds

Name            Speed   MTU  Mode     Summary

--  --------------  -----  ----  -------  --------------------------------

DN  BOND_TO_SPINES  N/A    9216  802.3ad  Bond Members: swp1(UP), swp2(UP)

cumulus@leaf2:mgmt:~$ net show interface bondmems

Name  Speed   MTU  Mode       Summary

--  ----  -----  ----  ---------  --------------------------

UP  swp1  1G     9216  LACP-UP    Master: BOND_TO_SPINES(DN)

UP  swp2  1G     9216  LACP-DOWN  Master: BOND_TO_SPINES(DN)

Verify bonds configuration:

Please note:

The bond interface is down because its peer (the MLAG peer) was not configured yet.

- Add the bond interface, BOND-TO-SPINES, and the host facing interfaces, swp8 and swp9, to the bridge:

$ nv set interface BOND_TO_SPINES bridge domain br_default

- Configure VLANs 2-3 and associate swp8 to VLAN2 and swp9 to VLAN3:

- $ nv set interface swp8 bridge domain br_default access 2

- $ nv set interface swp9 bridge domain br_default access 3

Task 2: Configuring MLAG – spine switches

- Access the spine switches and reset the configuration:

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine3:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.4/24 fe80::4638:39ff:fe00:20/64

cumulus@spine3:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

cumulus@spine4:mgmt:~$ ip -br a | grep eth0

eth0             UP             192.168.200.5/24 fe80::4638:39ff:fe00:22/64

cumulus@spine4:mgmt:~$ ip l | grep eth0

2: eth0: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 1500 qdisc pfifo_fast master mgmt state UP mode DEFAULT group default qlen 1000

Write down, on a side note, notepad or else, the IP addresses of the management ports, eth0. Those IP addresses will be configured as the MLAG backup-IPs.

- Also verify if / which vrf is being used.

Here: 192.168.200.4 and 192.168.200.5 (both vrf mgmt.)

- Configure the spine switches, spine3 (primary) and spine4 (secondary), as MLAG peers.

$ nv set mlag mac-address 44:38:39:FF:00:01 $ nv set mlag backup 192.168.200.4 vrf mgmt. $ nv set mlag init-delay 15 $ nv set mlag priority 2000 $ nv set interface peerlink bond member swp3-4

if you have selected your own IP address as the backup IP address, you will find the line:

Backup IP: 192.168.200.4 vrf mgmt (inactive: self)

indicating “inactive: self” as the reason.

A not yet reachable IP as the backup results in the following line:

Backup IP: 172.168.200.5 vrf mgmt (inactive)

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

cumulus@spine3:mgmt:~$ net show clag

The peer is not alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

Once you have configured both sides with a valid backup IP address, the show command will state "active", which is the desired outcome.

Please note:

- The init-delay is being changed for lab use, not a production best practice.
For production esp. for lagre scale deployments consult with your account team.

- Interfaces swp3 and swp4 will be configured as the MLAG peerlink.

- Switch spine3 is configured as the MLAG primary and spine4 as the secondary.

- On switch spine3 use spine4’s IP as the backup-IP and vice-versa.

- Configure bridge settings – VLANs and STP priority: on both spines:

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

cumulus@spine3:mgmt:~$ nv set bridge domain br_default vlan 2-3

cumulus@spine3:mgmt:~$ nv set bridge domain br_default stp priority 4096

- Apply the changes:

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

cumulus@spine3:mgmt:~$ nv config apply

Task 3: Configuring CLAG interfaces - spine switches

- Configure two CLAG interfaces on each of the spine switches.

- Interface ‘clag 1’ will aggregate swp1 on spine3 and swp2 on spine4 which are connected to switch leaf1

- Interface ‘clag 2’ will aggregate swp2 on spine3 and swp1 on spine4 which are connected to switch leaf2. For spine3:

$ nv set interface LEAF1 bond member swp1 $ nv set interface LEAF2 bond member swp2 $ nv set interface LEAF1-2 type bond $ nv set interface LEAF1 bond mlag id 1 $ nv set interface LEAF2 bond mlag id 2 $ nv set interface LEAF1-2 bridge domain br_default $ nv config apply

Working output:

cumulus@spine3:mgmt:~$ net show clag The peer is alive Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal) Backup IP: 192.168.200.5 vrf mgmt (active) System MAC: 44:38:39:ff:00:01 CLAG Interfaces Our Interface Peer Interface CLAG Id Conflicts Proto-Down Reason ---------------- ---------------- ------- -------------------- ----------------- LEAF1 LEAF1 1 - - LEAF2 LEAF2 2 - -

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

cumulus@spine4:mgmt:~$ nv config apply

Verify which interfaces have been added:

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

<SNIP>

auto peerlink

iface peerlink

bond-slaves swp3 swp4

bond-mode 802.3ad

bond-lacp-bypass-allow no

auto peerlink.4094

iface peerlink.4094

clagd-peer-ip linklocal

clagd-priority 1000

clagd-backup-ip 192.168.200.5 vrf mgmt

clagd-sys-mac 44:38:39:FF:00:01

clagd-args --initDelay 15

auto LEAF1

iface LEAF1

bond-slaves swp1

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 1

auto LEAF2

iface LEAF2

bond-slaves swp2

bond-mode 802.3ad

bond-lacp-bypass-allow no

clag-id 2

auto br_default

iface br_default

bridge-ports peerlink LEAF1 LEAF2

hwaddress 44:38:39:00:00:25

bridge-vlan-aware yes

bridge-vids 2 3

bridge-pvid 1

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

cumulus@spine3:mgmt:~$ nv config diff empty applied

- set:

bridge:

domain:

br_default:

stp:

priority: 4096

vlan:

'2': {}

'3': {}

mlag:

backup:

192.168.200.5:

vrf: mgmt

enable: on

init-delay: 15

mac-address: 44:38:39:FF:00:01

priority: 1000

system:

hostname: spine3

interface:

LEAF1:

bond:

member:

swp1: {}

mlag:

id: 1

LEAF1-2:

bond:

mlag:

enable: on

bridge:

domain:

br_default: {}

type: bond

LEAF2:

bond:

member:

swp2: {}

mlag:

id: 2

peerlink:

bond:

member:

swp3: {}

swp4: {}

type: peerlink

peerlink.4094:

base-interface: peerlink

type: sub

vlan: 4094

swp1-9:

type: swp

- View MLAG resulting configuration:

Verify which timers MLAG protocol is using by  : 
$ clagctl

Expected result:

“The peer is alive” and Spine3 is the primary, Spine4 the secondary with Priority values of 1000 and 2000 respectively.

An IPv6 LLA is used for the TCP connection between the peerlink.4094 sub-interfaces.

Both bonds (LEAF1 and LEAF2) are listed twice, once under “Our Interface” and once under the “Peer Interface” column.

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

cumulus@spine3:mgmt:~$ clagctl -v

The peer is alive

Our Priority, ID, and Role: 1000 44:38:39:00:00:01 primary

Peer Priority, ID, and Role: 2000 44:38:39:00:00:02 secondary

Peer Interface and IP: peerlink.4094 fe80::4638:39ff:fe00:2 (linklocal)

Backup IP: 192.168.200.5 vrf mgmt (active)

System MAC: 44:38:39:ff:00:01

CLAG Interfaces

Our Interface      Peer Interface     CLAG Id   Conflicts              Proto-Down Reason

----------------   ----------------   -------   --------------------   -----------------

LEAF1   LEAF1              1         -                      -

LEAF2   LEAF2              2         -                      -

<SNIP>

Timer Information

Timers             Value     Time remaining

----------------   -------   --------------

initDelay          15        00:00:00

peerTimeout        20        00:00:18

reloadTimer        300       00:00:00

sendTimeout        30

<SNIP>

The Backup IP is stated as (active)

Task 4: Servers’ IP settings

| VLAN ID | Server | ‘eth2’ IP Address |
| --- | --- | --- |
| VLAN 2 | host1 | 172.16.2.18/24 |
| VLAN 3 | host2 | 172.16.3.19/24 |
| VLAN 2 | host3 | 172.16.2.28/24 |
| VLAN 3 | host4 | 172.16.3.29/24 |

- Access the servers and configure an IP address for interface ‘eth2’ (see tables below).

Configure the server’s IP address and subnet mask

- Verify a static route entry to network 172.16.0.0/16 via the default gateway’s address.

Use the following table for default gateway assignment.

| VLAN ID | Default gateway address |
| --- | --- |
| vlan 2 | 172.16.2.254/24 |
| vlan 3 | 172.16.3.254/24 |

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.2.254

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.2.254    255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

$ sudo ip route add <NET_ADDRESS/MASK> dev <DEV> via <IP>

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

[cumulus@host1 ~]$ ping 172.16.2.28

PING 172.16.2.28 (172.16.2.28) 56(84) bytes of data.

64 bytes from 172.16.2.28: icmp_seq=1 ttl=64 time=2.60 ms

64 bytes from 172.16.2.28: icmp_seq=2 ttl=64 time=1.83 ms

Use ‘ping’ to check communication between servers in the same VLAN. 
For example, server host1 and host3 in VLAN2.

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine4:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

cumulus@spine3:mgmt:~$ net show bridge macs

VLAN      Master  Interface  MAC                TunnelDest  State      Flags  LastSeen

--------  ------  ---------  -----------------  ----------  ---------  -----  --------

2         bridge  LEAF1      44:38:39:00:00:11                                00:00:49

2         bridge  LEAF2      44:38:39:00:00:15                                00:00:52

3         bridge  LEAF1      44:38:39:00:00:13                                00:02:05

3         bridge  LEAF2      44:38:39:00:00:17                                00:02:05

Verify that the MLAG switches have their MAC address tables synchronized.

Please note:

- Once the MLAG configuration is completed, the spine switches appear as a single 
Layer 2 switch towards to Layer 2 network. Hence, they are capable to provide an efficient load balancing and better network utilization for the layer 2 network.

- In the following task additionally, the spine switches will be configured as VRR routers. Thus, they will provide default gateway redundancy and efficient load balancing towards the layer 3 network.

Task 5: Configuring VRR

Topology used in this task:

- Configure two SVIs (Switch Virtual Interfaces) on each of the spine switches, interface vlan 2 and interface vlan 3.

  - Configure an IP for each of the vlan interfaces.

  - Configure a VIP (Virtual IP) and a VMAC (Virtual MAC).

  - Use the following table for address assignment. Use /24 as the subnet mask.

| SWITCH SPINE3 |  |  |  |
| --- | --- | --- | --- |
| VLAN | SVI | VIP | VMAC |
| vlan 2 | 172.16.2.252/24 | 172.16.2.254/24 | 00:00:5e:00:01:02 |
| vlan 3 | 172.16.3.252/24 | 172.16.3.254/24 | 00:00:5e:00:01:03 |
| SWITCH SPINE4 |  |  |  |
| VLAN | SVI | VIP | VMAC |
| vlan 2 | 172.16.2.253/24 | 172.16.2.254/24 | 00:00:5e:00:01:02 |
| vlan 3 | 172.16.3.253/24 | 172.16.3.254/24 | 00:00:5e:00:01:03 |

$ nv set interface vlan2 ip address 172.16.2.252/24

$ nv set interface vlan2 ip vrr address 172.16.2.254/24

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

$ ip l

<SNIP>

13: vlan2@br_default: <BROADCAST,MULTICAST,UP,LOWER_UP> mtu 9216 qdisc noqueue state UP group default qlen 1000

link/ether 44:38:39:00:00:01 brd ff:ff:ff:ff:ff:ff

inet 172.16.2.252/24 scope global vlan2

valid_lft forever preferred_lft forever

inet6 fe80::4638:39ff:fe00:1/64 scope link

valid_lft forever preferred_lft forever

Verify VRR configuration:

The VRR interface is not listed because we are missing a parameter.

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

cumulus@spine3:mgmt:~$ nv set interface vlan2 ip vrr state up

cumulus@spine3:mgmt:~$ nv config apply

Invalid config

'interface vlan2 ip vrr' requires one of the following to be configured:

'system global anycast-id'

'system global anycast-mac'

'interface vlan2 ip vrr mac-id'

'interface vlan2 ip vrr mac-address'

$ nv set interface vlan2 ip vrr state up

$ nv set interface vlan2 ip vrr mac-address 00:00:5e:00:01:02

Repeat for vlan3 as well as for spine4.

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

[cumulus@host1 ~]$ ping 172.16.3.19

PING 172.16.3.19 (172.16.3.19) 56(84) bytes of data.

64 bytes from 172.16.3.19: icmp_seq=1 ttl=63 time=3.83 ms

64 bytes from 172.16.3.19: icmp_seq=2 ttl=63 time=1.90 ms

[cumulus@host1 ~]$ traceroute 172.16.3.19

traceroute to 172.16.3.19 (172.16.3.19), 30 hops max, 60 byte packets

1  172.16.2.254 (172.16.2.254)  1.511 ms  1.423 ms  1.435 ms

2  172.16.3.19 (172.16.3.19)  2.182 ms  1.994 ms *

Verify VRR operation.
Use ‘ping’ and ‘traceroute’ utilities to verify communication between hosts in different VLANs. 
For example, from ‘host1’ in VLAN 2 to ‘host2’ in VLAN 3.

- Verify MLAG/VRR failover.
Use continuous ‘ping’ between hosts in different VLANs. 
For example, from server ‘host1’ in VLAN 2 to server ‘host2’ in VLAN 3.

While ‘ping’ is running, reboot switch spine3. Was the traffic disrupted?

After switch spine3 reboots, reboot switch spine4. Was the traffic disputed now?

What are your conclusions regarding MLAG/VRR failover?

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

## PRACTICE 5: CONFIGURING BGP UNNUMBERED

Practice Objectives:

In this practice session you will configure BGP Unnumbered:

- Spine switches will be configured in the same AS and each of the leaf switches will be configured in its own AS.

- BGP unnumbered will be configured on all four switches.

- eBGP sessions will be established between the spine and leaf switches.

- Leaf switches will advertise their local IP prefixes.

- By the end of this practice session you will achieve end-to-end connectivity between all servers in your group, over the BGP Autonomous Systems.

Please note:

- Commands are demonstrated on switches selected switches. You should apply similar commands on the other two switches in your group.

Alternative lab:

You can configure BGP via the classical approach by using IPv4 Addresses on point-to-point links between the spines and leaf’s. For teaching purposes we use a /24 network but in production a /30 or /31 is often preferred.

Topology used in this practice session covering BGP unnumbered:

Task 1: Starting FRR

- Access the switches, reset the configuration, and set the host names:

# nv config apply empty

# nv set system hostname [leaf1|leaf2|spine3|spine4]

Task 2: Configuring loopback interfaces and enable interfaces between the spines and leafs:

| Switch | Loopback IP address |
| --- | --- |
| leaf1 | 172.16.100.1/32 |
| leaf2 | 172.16.100.2/32 |
| spine3 | 172.16.100.3/32 |
| spine4 | 172.16.100.4/32 |

- On each of the switches configure a loopback interface.

Use the following table for IP address assignment:

$ nv set interface lo ip address 172.16.100.1/32

$ nv config apply

Enable the interfaces between the spines and leaf’s as Layer 3 interfaces.

$ nv set interface swp1-2

$ nv config apply

Task 3: Configuring BGP Unnumbered

- Configure BGP unnumbered (the three ASN used are 65100, 65101, and 65102):

cumulus@leaf1:mgmt:~$ nv set router bgp autonomous-system 65101 cumulus@leaf1:mgmt:~$ nv set router bgp router-id 172.16.100.1 cumulus@leaf1:mgmt:~$ nv set vrf default router bgp neighbor swp1 remote-as external cumulus@leaf1:mgmt:~$ nv set vrf default router bgp neighbor swp2 remote-as external

- Advertise the loopback addresses and apply the changes.

cumulus@leaf1:mgmt:~$ nv set vrf default router bgp address-family ipv4-unicast network 172.16.100.1/32 cumulus@leaf1:mgmt:~$ nv config apply

After you configured the first switch for bgp you can verify the state of the FSM and the number (zero) of prefixes exchanged:

cumulus@leaf1:mgmt:~$ net show bgp sum show bgp ipv4 unicast summary ============================= BGP router identifier 172.16.100.1, local AS number 65101 vrf-id 0 BGP table version 1 RIB entries 1, using 200 bytes of memory Peers 2, using 46 KiB of memory Neighbor V AS MsgRcvd MsgSent TblVer InQ OutQ Up/Down State/PfxRcd PfxSnt swp1 4 0 0 0 0 0 0 never Idle 0 swp2 4 0 0 0 0 0 0 never Idle 0 Total number of neighbors 2

Please note:

Commands are demonstrated on one switche, all four switches should run and all loopback addresses should be known by all switches (one local and three remote).

- Once you have configured all four devices you can verify configuration via net show commands or switch to the FRR provided CLI:

$ sudo vtysh $ show run $ show ip bgp sum

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

spine4# show ip bgp summary

IPv4 Unicast Summary:

BGP router identifier 172.16.100.4, local AS number 65100 vrf-id 0

BGP table version 3

RIB entries 5, using 1000 bytes of memory

Peers 2, using 46 KiB of memory

Neighbor        V         AS   MsgRcvd   MsgSent   TblVer  InQ OutQ  Up/Down State/PfxRcd   PfxSnt

leaf2(swp1)     4      65102         7         7        0    0    0 00:00:04            1        3

leaf1(swp2)     4      65101         9         8        0    0    0 00:00:07            1        3

Total number of neighbors 2

you should see two neigbors and a number !=0  in the PfxSnt column

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

spine4# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

B>* 172.16.100.1/32 [20/0] via fe80::4638:39ff:fe00:c, swp2, weight 1, 00:01:52

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:a, swp1, weight 1, 00:01:49

C>* 172.16.100.4/32 is directly connected, lo, 00:01:56

You can also verify the RIB via “show ip route” within the FRR CLI

Verify loopback prefixes are reachable:

Please note:

- NEXT-HOP is the neighbor’s IPv6 Link Local Address

- BGP multipath is enabled

Task 4: Advertise local IP prefixes

- On the leaf switches, assign IP addresses to the host-facing interfaces,

swp8 and swp9.
Advertise the IP prefixes in the BGP process.
Use the IP addresses listed in the below topology. Use 24 bits for the subnet mask.

Example for leaf1:

$ nv set interface swp8 ip address 172.16.18.1/24 $ nv set interface swp9 ip address 172.16.19.1/24 $nv set vrf default router bgp address-family ipv4-unicast network 172.16.18.0/24 $nv set vrf default router bgp address-family ipv4-unicast network 172.16.19.0/24 $nv config apply

- Verify that the remote leaf switch has learned the IP prefixes:

- Switch leaf1 should have in its routing table the IP prefixes advertise by switch leaf2:

  - 172.16.28.0/24

  - 172.16.29.0/24

Intermediate state after configuring the first leaf, you see the local networks in the 
BGP Topology Table:

cumulus@leaf1:mgmt:~$ net show bgp show bgp ipv4 unicast ===================== BGP table version is 9, local router ID is 172.16.100.1, vrf id 0 Default local pref 100, local AS 65101 Status codes: s suppressed, d damped, h history, * valid, > best, = multipath, i internal, r RIB-failure, S Stale, R Removed Nexthop codes: @NNN nexthop's vrf id, < announce-nh-self Origin codes: i - IGP, e - EGP, ? - incomplete Network Next Hop Metric LocPrf Weight Path *> 172.16.18.0/24 0.0.0.0 0 32768 i *> 172.16.19.0/24 0.0.0.0 0 32768 i *> 172.16.100.1/32 0.0.0.0 0 32768 i Displayed 3 routes and 3 total paths

Switch leaf2 should have in its routing table the IP prefixes advertise by switch leaf1:

- 172.16.18.0/24

- 172.16.19.0/24

$ net show route bgp

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

leaf1# show ip route

Codes: K - kernel route, C - connected, S - static, R - RIP,

O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP,

T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP,

F - PBR, f - OpenFabric,

> - selected route, * - FIB route, q - queued, r - rejected, b - backup

t - trapped, o - offload failure

C>* 172.16.18.0/24 is directly connected, swp8, 00:06:06

C>* 172.16.19.0/24 is directly connected, swp9, 00:06:06

B>* 172.16.28.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

B>* 172.16.29.0/24 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:00:17

*                       via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:00:17

C>* 172.16.100.1/32 is directly connected, lo, 2d18h27m

B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:50:33

*                        via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:50:33

B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 2d18h12m

B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:53:51

Task 5: Verify end-to-end connectivity

- Configure the servers IP settings. Configure an IP address and a subnet mask for interface ’eth2’. Use the following tables for IP address assignment. 
Use /24 as the subnet mask.

| Server | ’eth2’ IP Address | Next Hop to 172.16.0.0/16 |
| --- | --- | --- |
| host1 | 172.16.18.2 | 172.16.18.1 |
| host2 | 172.16.19.2 | 172.16.19.1 |
| host3 | 172.16.28.2 | 172.16.28.1 |
| host4 | 172.16.29.2 | 172.16.29.1 |

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

cumulus@host1:~$ sudo ip route add 172.16.0.0/16 dev eth2 via 172.16.18.1

Add a route entry to 172.16.0.0/16 via interface ’eth2’:
$ sudo ip route add <ADDRESS/MASK> dev <DEV> via <IP_ADDRESS>

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

cumulus@host1:~$ route

Kernel IP routing table

Destination     Gateway         Genmask         Flags Metric Ref    Use Iface

default         _gateway        0.0.0.0         UG    0      0        0 eth0

172.16.0.0      172.16.18.1     255.255.0.0     UG    0      0        0 eth2

172.16.2.0      0.0.0.0         255.255.255.0   U     0      0        0 eth2

192.168.200.0   0.0.0.0         255.255.255.0   U     0      0        0 eth0

Or via netplan, for example:

<SNIP>

routes:

- to: 172.16.0.0/16

via: 172.16.18.1

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# traceroute  172.16.28.2

traceroute to 172.16.28.2 (172.16.28.2), 30 hops max, 60 byte packets

1  172.16.18.1  (172.16.18.1)  0.761 ms  0.927 ms  0.899 ms

2  172.16.100.3 (172.16.100.3) 1.433 ms  1.354 ms  1.372 ms

3  172.16.100.2 (172.16.100.2) 2.449 ms  2.058 ms  2.493 ms

4  172.16.28.2  (172.16.28.2)  3.215 ms  3.244 ms  3.348 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

[cumulus@host1 ~]# ping 172.16.28.2

PING 172.16.28.2 (172.16.28.2) 56(84) bytes of data.

64 bytes from 172.16.28.2: icmp_seq=1 ttl=61 time=4.04 ms

64 bytes from 172.16.28.2: icmp_seq=2 ttl=61 time=2.56 ms

Use ‘ping’ and ‘traceroute’ utilities to verify communication between servers in different Autonomous Systems. 

For example, ping from server ‘host1’ (AS 65101) to server ‘host3’ (in AS 65102).

Please note:

The next practice relies on BGP Unnumbered configuration. Please make sure to save the configuration before exiting.

$ nv config save

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

## PRACTICE 6: CONFIGURING VXLAN WITH EVPN

Practice Objectives:

In this practice session you will configure VXLAN with EVPN:

- The leaf switches will be configured as the VTEPs

- EVPN will be configured as the VXLAN control plane

- Two VXLAN Network IDs (VNIs) will be configured:

  - VNI-10 will connect VLAN 10 on switch leaf1 and VLAN 100 on switch leaf2.

  - VNI-20 will connect VLAN 20 on switch leaf1 and VLAN 200 on switch leaf2.

- Hosts in each VNI will be able to communicate in layer 2 over the underlay layer 3 network.

Please note:

The configuration in this practice session relies on the previous practice. 
Make sure that BGP Unnumbered is properly configured and fully operational.

Topology used in this practice session:

Task 1: Configuring servers IP settings

- Configure an IP address and a subnet mask for interface ’eth2’. 
Use the following tables for IP address assignment. Use /24 as the subnet mask.

| Server | ETH2 IP Address |
| --- | --- |
| host1 | 172.16.10.18 |
| host2 | 172.16.20.19 |
| host3 | 172.16.10.28 |
| host4 | 172.16.20.29 |

- Clear existing IP configuration:
$ sudo ifconfig <DEV> 0.0.0.0

- Configure an IP address and a subnet mask:
$ sudo ifconfig <DEV< <IP/MASK>

Alternatively consider to use Netplan to configure your Linux hosts[1-4].

Task 2: Configuring a Bridge with two VLANs and access-ports

- Access the leaf switches and configure the host-facing ports swp8 and swp9. 
First clear any existing configuration, then assign the ports to the appropriate VLAN:

  - Switch leaf1 - assign swp8 to VLAN 10 and swp9 to VLAN 20

  - Switch leaf2 - assign swp8 to VLAN 100 and swp9 to VLAN 200

$ nv unset interface swp8

$ nv unset interface swp9

$ nv config apply

Interim state, interfaces swp8-9 are unconfigured:

cumulus@leaf1:mgmt:~$ net show int State Name Spd MTU Mode LLDP Summary ----- ---- --- ----- ------------ ---- -------------------------- UP lo N/A 65536 Loopback IP: 127.0.0.1/8 lo IP: 172.16.100.1/32 lo IP: ::1/128 UP eth0 1G 1500 Mgmt Master: mgmt(UP) eth0 IP: 192.168.200.2/24(DHCP) UP swp1 1G 9216 Default UP swp2 1G 9216 Default UP mgmt N/A 65536 VRF IP: 127.0.0.1/8 mgmt IP: ::1/128

Leaf1:

$ nv set interface swp8 bridge domain br_default access 10

$ nv set interface swp9 bridge domain br_default access 20

$ nv set bridge domain br_default vlan 10,20

$ nv config apply

cumulus@leaf1:mgmt:~$ net show int State Name Spd MTU Mode LLDP Summary ----- ---------- --- ----- --------- ------------------------- -------------------------- UP lo N/A 65536 Loopback IP: 127.0.0.1/8 lo IP: 172.16.100.1/32 lo IP: ::1/128 UP eth0 1G 1500 Mgmt Master: mgmt(UP) eth0 IP: 192.168.200.2/24(DHCP) UP swp1 1G 9216 Default UP swp2 1G 9216 Default UP swp8 1G 9216 Access/L2 host1 (44:38:39:00:00:11) Master: br_default(UP) UP swp9 1G 9216 Access/L2 Master: br_default(UP) UP br_default N/A 9216 Bridge/L2 UP mgmt N/A 65536 VRF IP: 127.0.0.1/8 mgmt IP: ::1/128

Leaf2:

$ nv set interface swp8 bridge domain br_default access 100 $ nv set interface swp9 bridge domain br_default access 200 $ nv set bridge domain br_default vlan 100,200 $ nv config apply

Verify the bridge configuration on both leafs:

$ bridge vlan

$ bridge link

Task 3: Configuring VNIs

- Access the leaf switches and configure the VNIs:

  - Create a VXLAN interface (NVE) and use the leaf’s loopback IP address as 
its source

  - Map the VXLAN to a VLAN 
Leaf1:	VLAN 10 : VXLAN 10
		VLAN 20 : VXLAN 20
Leaf2:	VLAN 100 : VXLAN 10
		VLAN 200 : VXLAN 20

Leaf1:

$ nv set nve vxlan source address 172.16.100.1 $ nv set bridge domain br_default vlan 10 vni 10 $ nv set bridge domain br_default vlan 20 vni 20 $ nv config apply

Leaf2:

$ nv set nve vxlan source address 172.16.100.2 $ nv set bridge domain br_default vlan 100 vni 10 $ nv set bridge domain br_default vlan 200 vni 20 $ nv config apply

Task 4: Configuring EVPN

- Access the switches and configure EVPN.

$ nv set evpn enable on $ nv set vrf default router bgp address-family l2vpn-evpn enable on $ nv set vrf default router bgp neighbor swp1 address-family l2vpn-evpn enable on $ nv set vrf default router bgp neighbor swp2 address-family l2vpn-evpn enable on $ nv config apply

- Verify MAC VXLAN Information.

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

leaf1# show evpn mac vni all

VNI 10 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:11 local        swp8                           10    0/0

44:38:39:00:00:15 remote       172.16.100.2                         0/0

VNI 20 #MACs (local and remote) 2

Flags: B=bypass N=sync-neighs, I=local-inactive, P=peer-active, X=peer-proxy

MAC               Type   Flags Intf/Remote ES/VTEP            VLAN  Seq #'s

44:38:39:00:00:13 local        swp9                           20    0/0

44:38:39:00:00:17 remote       172.16.100.2                         0/0

$ show evpn mac vni all

cumulus@leaf1:mgmt:~$ net show bridge macs VLAN Master Interface MAC TunnelDest State Flags LastSeen -------- ---------- ---------- ----------------- ------------ --------- ------------------ -------- 10 br_default swp8 44:38:39:00:00:11 00:00:02 10 br_default vxlan48 44:38:39:00:00:15 extern_learn 00:13:52 20 br_default swp9 44:38:39:00:00:13 00:00:20 20 br_default vxlan48 44:38:39:00:00:17 extern_learn 00:13:52 untagged vxlan48 00:00:00:00:00:00 172.16.100.2 permanent self 00:13:52 untagged vxlan48 00:00:00:00:00:00 172.16.100.2 permanent self 00:13:52 untagged vxlan48 44:38:39:00:00:15 172.16.100.2 self, extern_learn 00:13:52 untagged vxlan48 44:38:39:00:00:17 172.16.100.2 self, extern_learn 00:13:52 untagged br_default br_default 44:38:39:00:00:23 permanent 00:52:08 untagged br_default swp8 44:38:39:00:00:12 permanent 00:52:09 untagged br_default swp9 44:38:39:00:00:14 permanent 00:52:09 untagged br_default vxlan48 86:f4:84:2a:61:26 permanent 00:47:57

Task 5: Verify end-to-end communication

- Each VNI is a logical layer 2 network over the underlay layer 3 network.
Logically there are no layer 3 hops between hosts in the same VNI.

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# ping 172.16.10.28

PING 172.16.10.28 (172.16.10.28) 56(84) bytes of data.

64 bytes from 172.16.10.28: icmp_seq=1 ttl=64 time=7.19 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

[cumulus@host1 ~]# traceroute 172.16.10.28

traceroute to 172.16.10.28 (172.16.10.28), 30 hops max, 60 byte packets

1  172.16.10.28 (172.16.10.28)  4.887 ms  4.817 ms  4.788 ms

Use ‘ping’ and ‘traceroute’ utilities to verify end-to-end communication between hosts in a VNI. For example, from host ‘host1’ to ‘host3’ that are configured in VNI-10.

Please note: 
The next practice relies on VXLAN with EVPN configuration. Please make sure to save the configuration before exiting.

## PRACTICE 7: DISTRIBUTED SYMMETRIC VXLAN ROUTING

## PRACTICE 7: DISTRIBUTED SYMMETRIC VXLAN ROUTING

Practice Objectives:

In this practice session you will configure distributed symmetric VXLAN routing.

- VTEPs (leaf switches) will be configured as the distributed anycast default gateway, i.e., each VTEP will be configured with a VIP and VMAC for each of the subnets.
Each VTEP will serve as the default gateway for its locally connected hosts.

- In symmetric VXLAN routing both the ingress and egress VTEPs perform the routing.

- A new special transit VNI is used for all routed VXLAN traffic, called the L3-VNI.

Please note:

The configuration in this practice session relies on the previous practice. Make sure that BGP Unnumbered and VXLAN with EVPN are properly configured and fully operational. Test and verify that hosts in the same VNI can communicate with each other.

As the FHRP we will use VRR under our SVI’s.

- set: router: vrr: enable: on interface: vlan10: ip: address: 172.16.10.252/24: {} vrr: address: 172.16.10.254/24: {} enable: on mac-address: 00:00:00:00:00:0A state: up: {} type: svi vlan: 10 Adresses: Leaf1: SVI: VLAN10 172.16.10.252/24 VRR: 172.16.10.254/24 MAC 00:00:00:00:00:0A

Leaf2:

SVI: VLAN100 172.16.10.253/24 VRF: NVIDIA VRR: 172.16.10.254/24 MAC 00:00:00:00:00:0A SVI: VLAN200 172.16.20.253/24 VRF: NVIDIA VRR: 172.16.20.254/24 MAC 00:00:00:00:00:0B

The L3-VNI construct will use the VRF named NVIDIA along with the VXLAN ID of 4001 on both leaf’s.

To verify L3 Distributed Symmetric the SVI on leaf1 for vlan 20 and the vlan 20 will need to be absent. Please remove them if they are present.

Topology used in this practice session:

Task 1: Configuring the L3-VNI

- Access both leaf switches and configure a per-tenant VXLAN interface.
Use VNI 4001.

$ nv set vrf NVIDIA evpn vni 4001 $ nv set interface vlan10 ip vrf NVIDIA $ nv config apply

Question:
Why is the subnet 172.16.10.0/24 on leaf1 not shown in the routing table?

cumulus@leaf1:mgmt:~$ net show route show ip route ============= Codes: K - kernel route, C - connected, S - static, R - RIP, O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP, T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP, F - PBR, f - OpenFabric, > - selected route, * - FIB route, q - queued, r - rejected, b - backup t - trapped, o - offload failure C>* 172.16.100.1/32 is directly connected, lo, 06:48:17 B>* 172.16.100.2/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:01:40 * via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:01:40 B>* 172.16.100.3/32 [20/0] via fe80::4638:39ff:fe00:5, swp1, weight 1, 00:01:43 B>* 172.16.100.4/32 [20/0] via fe80::4638:39ff:fe00:b, swp2, weight 1, 00:01:40

Please note:

Switch leaf2 should be configured similarly but for vlan 100 and 200.

Please verify the uniqueness of the system-mac addresses between leaf1 and leaf2:

cumulus@leaf1:mgmt:~$ cat /run/system_mac 44:38:39:00:00:23 cumulus@leaf2:mgmt:~$ cat /run/system_mac 44:38:39:00:00:25

In case the MACs are equal, please inform your instructor.

Task 3: Verify L3-VNI configuration

leaf1# show evpn vni

VNI        Type VxLAN IF              # MACs   # ARPs   # Remote VTEPs  Tenant VRF

10         L2   vxlan48               2        4        1               NVIDIA

4001       L3   vxlan99               1        1        n/a             NVIDIA

leaf2# sh evpn vni

VNI        Type VxLAN IF              # MACs   # ARPs   # Remote VTEPs  Tenant VRF

10         L2   vxlan48               2        4        1               NVIDIA

20         L2   vxlan48               1        3        0               NVIDIA

4001       L3   vxlan99               1        1        n/a             NVIDIA

leaf1# show evpn vni

VNI        Type VxLAN IF              # MACs   # ARPs   # Remote VTEPs  Tenant VRF

10         L2   vxlan48               2        4        1               NVIDIA

4001       L3   vxlan99               1        1        n/a             NVIDIA

leaf2# sh evpn vni

VNI        Type VxLAN IF              # MACs   # ARPs   # Remote VTEPs  Tenant VRF

10         L2   vxlan48               2        4        1               NVIDIA

20         L2   vxlan48               1        3        0               NVIDIA

4001       L3   vxlan99               1        1        n/a             NVIDIA

leaf1# show evpn vni detail

VNI: 10

Type: L2

Vlan: 0

Bridge: br_default

Tenant VRF: NVIDIA

VxLAN interface: vxlan48

VxLAN ifIndex: 22

Local VTEP IP: 172.16.100.1

Mcast group: 0.0.0.0

Remote VTEPs for this VNI:

172.16.100.2 flood: HER

Number of MACs (local and remote) known for this VNI: 2

Number of ARPs (IPv4 and IPv6, local and remote) known for this VNI: 4

Advertise-gw-macip: No

Advertise-svi-macip: No

VNI: 4001

Type: L3

Tenant VRF: NVIDIA

Vlan: 0

Bridge: -

Local Vtep Ip: 172.16.100.1

Vxlan-Intf: vxlan99

SVI-If: vlan444_l3

State: Up

VNI Filter: none

System MAC: 44:38:39:00:00:23

Router MAC: 44:38:39:00:00:23

L2 VNIs: 10

leaf1# show evpn vni detail

VNI: 10

Type: L2

Vlan: 0

Bridge: br_default

Tenant VRF: NVIDIA

VxLAN interface: vxlan48

VxLAN ifIndex: 22

Local VTEP IP: 172.16.100.1

Mcast group: 0.0.0.0

Remote VTEPs for this VNI:

172.16.100.2 flood: HER

Number of MACs (local and remote) known for this VNI: 2

Number of ARPs (IPv4 and IPv6, local and remote) known for this VNI: 4

Advertise-gw-macip: No

Advertise-svi-macip: No

VNI: 4001

Type: L3

Tenant VRF: NVIDIA

Vlan: 0

Bridge: -

Local Vtep Ip: 172.16.100.1

Vxlan-Intf: vxlan99

SVI-If: vlan444_l3

State: Up

VNI Filter: none

System MAC: 44:38:39:00:00:23

Router MAC: 44:38:39:00:00:23

L2 VNIs: 10

Verify that the L3-VNI is configured:

leaf1# show ip route vrf NVIDIA Codes: K - kernel route, C - connected, S - static, R - RIP, O - OSPF, I - IS-IS, B - BGP, E - EIGRP, N - NHRP, T - Table, v - VNC, V - VNC-Direct, A - Babel, D - SHARP, F - PBR, f - OpenFabric, > - selected route, * - FIB route, q - queued, r - rejected, b - backup t - trapped, o - offload failure VRF NVIDIA: K>* 0.0.0.0/0 [255/8192] unreachable (ICMP unreachable), 01:45:23 C * 172.16.10.0/24 [0/1024] is directly connected, vlan10-v0, 01:45:23 C>* 172.16.10.0/24 is directly connected, vlan10, 01:45:23 B>* 172.16.10.28/32 [20/0] via 172.16.100.2, vlan444_l3 onlink, weight 1, 00:07:33 B>* 172.16.20.29/32 [20/0] via 172.16.100.2, vlan444_l3 onlink, weight 1, 00:07:33

Task 4: Test inter-VXLAN communication

cumulus@host1:~$ traceroute 172.16.20.29

traceroute to 172.16.20.29 (172.16.20.29), 30 hops max, 60 byte packets

1  172.16.10.252 (172.16.10.252)  0.588 ms  0.546 ms  0.546 ms

2  172.16.20.253 (172.16.20.253)  2.656 ms  2.653 ms  2.652 ms

3  172.16.20.29 (172.16.20.29)  3.431 ms  3.415 ms  3.416 ms

cumulus@host1:~$ traceroute 172.16.20.29

traceroute to 172.16.20.29 (172.16.20.29), 30 hops max, 60 byte packets

1  172.16.10.252 (172.16.10.252)  0.588 ms  0.546 ms  0.546 ms

2  172.16.20.253 (172.16.20.253)  2.656 ms  2.653 ms  2.652 ms

3  172.16.20.29 (172.16.20.29)  3.431 ms  3.415 ms  3.416 ms

cumulus@host1:~$ ping 172.16.20.29

PING 172.16.20.29 (172.16.20.29) 56(84) bytes of data.

64 bytes from 172.16.20.29: icmp_seq=7 ttl=62 time=2.91 ms

64 bytes from 172.16.20.29: icmp_seq=8 ttl=62 time=3.44 ms

Use ‘ping’ and ‘traceroute’ utilities to verify end-to-end communication between hosts in different VNIs. 
For example, ping from host ‘host1’ to ‘host3 and host4’
In this case the egress switch leaf1 will perform routing between the source VNI and the L3-VNI, while the egress switch leaf2 will perform routing between L3-VNI and the destination VNI.